###

Date: 2025-12-01

###

Document version: 3.2

# Document Control

## Document Information

|  | Information |
| --- | --- |
| Document Name | QKD MACsec |
| Document Owner | Juniper Professional Services |
| Issue Date | 2025-12-01 |
| Last Saved Date | 2025-11-27 |
| File Name | QKD_MACsec |

## Document History

| Version | Issue Date | Changes |
| --- | --- | --- |
| 3.2 | 2025-12-01 | Document Main |

## Document Approvals

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Programme Director | William Mead |  |  |
| Programme Manager | Mark Tickner |  |  |
| PS Consultant | Andrea Terren |  |  |

## Contribution

For contributions, bug reports, or feature requests, please contact the development team at quantum_computing_team@juniper.net.

## License

Copyright (©) Juniper Networks, Inc., 2025. All right Reserved.

## License

This project is open-source and licensed under the MIT License – see the LICENSE file for details. If you would like to use this product under a different license, kindly contact us on juniper.net.

## Acknowledgements

Thank you for using the MACSEC Configuration Script! For more information, please refer to the documentation. This software has been developed in the projects …

# MACsec overview

It’s an industry standard protocol defined in IEEE 802.1AE (Ethertype 0x88E5, supporting both GCM/AES/128 and GCM/AES/256). MACsec offers line-rate layer 2 hardware based encryption on a hop-by-hop basis. MACsec is supported on MX, PTX, and ACX series routers. MACsec is also supported on some EX and QFX series switches.

Please refer to the Juniper MACsec support online documentation for current Juniper Networks hardware and JunoOS software supporting MACsec.

MACsec is best used to secure device-to-device connections in a colocation center or a data center environment, building-to-building links in a city, or even city-to-city links traversing long distances including undersea or terrestrial cables. You can also encrypt Layer 2 connections via a provider WAN with MACsec, performing “hop-by-hop” encryption/decryption across multiple tunnels.

MACsec is enabled on a “per-interface” basis. Once enabled on an interface, it drops all frames except MACsec encrypted packets. However, you can configure MACsec Should-secure (Fail Open Mode, default) to allow for unprotected traffic if MACsec negotiation should fail.

## PQC and MACsec integration

Post-quantum cryptography (PQC) is designed to be resistant against attacks from quantum computers. Quantum computers have the potential to break many of the current cryptographic systems that are widely used today, including RSA and ECC (Elliptic Curve Cryptography).

PQC aims to provide an alternative to these cryptographic systems that is secure against quantum attacks.

This documentation describes the implementation of MACsec together with quantum key distribution (QKD) networks.

Automated deployment of MACsec creates secure channel, generates keying material, and configures the routers/switches for each detected link between them. The script detects link changes and performs rekeying to provide a secure, configuration-free operation of MACsec.

## Terminology and Entities involved

- MACsec Key Agreement Protocol (MKA): Used to discover MACsec capable peers and used to negotiate encryption keys (for data encryption and for SAK encryption) - IEEE 802.1X-REV. MACsec security keys and sessions are managed by the MKA protocol software stack running on the Junos OS.This protocol is defined in IEEE standard 802.1X (specifically 802.1X-2010)

- Connectivity Association (CA): Defines a secure relashionship between MACsec capable peers. It is a set of MACsec attributes that are used by interfaces to create inbound and outbound MACsec secure channels (or connections in simple terms) through which encrypted bi-directional traffic flows.

- Programming the key involves two items, CKN and CAK:

  - Connectivity Association Key (CAK): static or dynamic key exchanged by MACsec speakers, this can be seen as a “primary key” that is used to derive all other session keys

  - Connectivity Association Key Name (CKN): any name that defines a CAK

- Primary and Fallback Keys: primary key is used to negotiate an MKA; if this fails the fallback key is used

- Security Association Key (SAK): derived from CAK used to encrypt data

- Key Server: generates SAK

A useful way to think of a MACsec-protected link is to split it into a control plane and a data plane. The picture below illustrates the MKA control plane and the actual user data with MACsec wrappers, the data plane.

MACsec User and Data Planes

A connectivity association (CA) is a common MACsec relationship between all devices on a LAN who share a common key. In a MACsec association with two devices, a CA contains two unidirectional secure channels, one from each device. Finally, each secure channel has a series of SAs. In Junos OS, SAs last the shorter of either the time until the next key rollover in the chain or the wrap of the packet sequence counter. The operator configures the CAK, which secures the CA and from which Junos OS derives the SAK, which in turn secures each SA.

The initial IEEE MACsec standard (IEEE 802.1AE) describes the data plane functions and excluded key management. An additional IEEE specification (802.1X 2010) provides the mechanism for devices to leverage the 802.1X specification to manage keys between devices.

## Static CAK

You initially establish a MACsec secured link using a pre-shared key (PPK) when you are using static CAK security mode to enable MACsec. A pre-shared key includes a connectivity association name (CKN) and its own CAK. The CKN and CAK are configured by the user in the connectivity association and must match on both ends of the link to initially enable MACsec.

Once matching pre-shared keys are successfully exchanged, the MACsec Key Agreement (MKA) protocol is enabled. The MKA protocol is responsible for maintaining MACsec on the link, and decides which switch on the point-to-point link becomes the key server. The key server then creates an SAK that is shared with the switch at the other end of the point-to-point link only, and that SAK is used to secure all data traffic traversing the link. The key server will continue to periodically create and share a randomly-created SAK over the point-to-point link for as long as MACsec is enabled.

# Core concepts of the python MACsec/QKD script

The quantum key distribution (QKD) MACSEC script is written in python3. The script allows the JUNOS devices to fetch key material from QKD/KME and update the MACsec static CAK accordingly.

The script makes use of the libraries in the ETSI GS QKD 014 v1.1.1 - Reference Implementation public repository.

## Release Notes for MACSEC Configuration Script

This release introduces significant improvements and additional functionality to the MACSEC Configuration Script. The script is designed for managing MACSEC configurations on Junos devices, fetching keys, and handling device settings with enhanced features, improved error handling, and more robust performance.

### New Features

- JSON/YAML inventories: for parametric metadata, including:

- certificates (pub/pri keys, RootCA)

- QKD Keys/key-IDs

- Provisioning of the basic MACsec configs applicable to different Juniper devices supporting MACsec, using config-set-commands and parametric data (i.e. keys, key-id, profile names, etc.)

- REST API response: calls are applied (instead of curl) commands for:

- Verifications of HTTP/HTTPs return messages codes (4xx, 5xx)

- syslog and trace messages (with timer profiles) upon return codes

- Handling exceptions/errors

- Success code verification and telemetry/monitoring after QKD successful setup

- Enhanced Threading: Improved the handling of concurrent device processing with dynamic thread management.

- Configuration Validation: Added validation for configuration files to ensure consistency and prevent errors.

- Improved Error Handling: Enhanced error handling and logging for better diagnostics and debugging.

- Password Encryption: Incorporated encryption for sensitive information such as passwords in the configuration dictionary.

- Extended Logging Options: Added additional logging options for detailed trace and debug information.

- Scaling Tests: Implemented scaling tests to handle multiple interfaces more efficiently.

### Enhancements

- get_args():

  - Added support for additional command-line arguments.

  - Improved argument parsing and validation.

- initialize_logging(args):

  - Enhanced logging setup to support multiple log levels and output formats.

  - Added support for logging to both console and file outputs.

- check_and_apply_initial_config(dev, targets_dict, log):

  - Improved the logic for applying initial configurations, including additional checks and optimizations.

  - Added more robust error handling and validation of configuration parameters.

- process(dev, log):

  - Enhanced key fetching logic to handle different scenarios and improve performance.

  - Added detailed logging and debug output for better visibility into the processing steps.

- fetch_kme_key(session, local_name, log, remote_mnmgt_add, kme_url, key_id=None):

  - Improved error handling for key fetching operations.

  - Added support for additional parameters and configurations.

- save_key_ids(key_dict):

  - Added support for encrypted storage of key IDs.

  - Improved file handling and error management.

### Script Configuration

- targets_dict: The dictionary that has to be updated will the following information consumed by the script:

  - Updated dictionary containing device-specific configurations with encrypted passwords.

  - device specific information like: ip, interfaces (that forms the MacSec channel), associated KME (dns name and ip)

  - CA Server information like: secrets, ip, CA certificate path, names

  - QKD/KME devices roles: master, slave and additional slave (assumed that the QKD/KME devices provider supports additional_slave_SAE_IDs parameter in the case of more than 2 devices used in the MacSec chains. -> please refer to the Quantum Key Distribution - Protocol and data format of REST-based key delivery API)

  - system information like:

    - maxthreads (in case of off-box multithreading execution)

    - event-options … <start-time start time>

- config.ini: Optional configuration file for additional settings and parameters.

### Script Usage

- Offbox

  - Command-Line Arguments:

    - targets: List of target hosts to process.

    - -t, --threads: Number of threads to use for concurrent processing.

    - -v, --verbose: Increase verbosity level for logging.

    - -t, --trace: Dump debug-level logs to a trace.log file.

    - --config-file: Specify a configuration file for additional settings.

  - Manual qkd_v3.2.0.py Script Execution - Example Commands:

    - Run Script with Default Settings from the targets_dict (see the section Configuration):

    - python3 qkd_v3.2.0.py

    - Run Script with Specified Number of Threads:

    - python3 qkd_v3.2.0.py -t 4

    - Run Script with Verbose Logging:

    - python3 qkd_v3.2.0.py --verbose

    - Run Script with Trace Logging:

    - python3 qkd_v3.2.0.py --trace

  - Scheduled qkd_v3.2.0.py Script Execution using Crontab:

  - Users’ crontab files are named according to the user’s name, and their location varies by operating systems. In Red Hat based distributions such as CentOS, crontab files are stored in the /var/spool/cron directory, while on Debian and Ubuntu files are stored in the /var/spool/cron/crontabs directory. Although you can edit the user crontab files manually, it is recommended to use the below crontab command (will run the script every 10 minutes): bash   # crontab -u <user_name> -e   */10 * * * * python3 qkd_v3.2.0.py

- Onbox

  - Import the qkd_v3.2.0.py and profiling Profile python scripts under /var/db/scripts/event/qkd_v3.2.0.py

  - Manually run the script one time from the device and it will apply the needed even-options configuration or manually configure the event-options on the device (see Event-options section) and the script will start running every <time-interval seconds> starting with <start-time start-time>.

### Error Handling

- Logging: Improved logging for capturing detailed error and debug information.

- Exceptions: Enhanced exception handling for robustness and reliability during device processing and configuration.

### Known/Potential Issues/Limitations

- Scaling Performance: While scaling tests have been implemented, performance optimizations for very large configurations may still be required.

- renew_certificates function: Current version of this function is not running with onbox execution of the script because pyOpenSSL is not part of the Junos python3 modules.

### Future Improvements

- Advanced Configuration Management: Implement more advanced configuration management features.

- Extended Encryption Support: Enhance support for encryption and security features.

- Enhanced User Interface: Develop a user-friendly interface for easier configuration and management.

- OUI Ethertype 88B7 Key-ID tranport: tranport key-ids through OUI Ethernet-type instead of LLDP or scp protocol (given the key-id remains the same for the whole chain of MACSEC tunnels).

- Enhanced PPK: Static vs dynamic Pre-Shared Key (PPK), represented with two strings separated by spaces. The first string is PPK_ID and the second one is a path to another file which the PPKs will be taken from. This file should also have only two strings in it. The first one would be offset and the second one a long string that will be used as a one-time pad.

- Vault container(SSSD): Use of Vault container (e.g. SSSD) for key material received from KME quantum servers. This requires extra server or VM (close to SAE device) between end SAE and KME itself. If extra server isn’t an option, this should be directly embedded into Junos Code using Junos to trigger API request/response from KME to SAE network device

## Script details

The script is supposed to be kept at /var/db/scripts/event/.py, and it can be scheduled using configuration “event-options” in JunOS. The current version of the script is 3.2.0 and the user is harcoded as “lab”.

The script discriminates if it’s going to be run “on-box” or “off-box”, i.e. whether it’s going to be running via “event python scripting” installed into the Juniper devices (e.g. an MX device) or if it’s going to manage keys retrieval within an external device (likely to be installed within the KME itself or on a separate Linux target host).

The script sets a number of parameters to establish TLS connectivity with a RootCA Certification Authority. Key material is exchanged via TLS between the KME and SAE using the KME available REST APIs (depending on the 3rd party QKD vendor compute appliance, different REST APIs might be available to use). The ETSI 014 QKD REST API is a specification developed by the European Telecommunications Standards Institute (ETSI) for the management of QKD networks. This API provides a standardized interface for communication between different QKD network components, such as QKD devices, key servers, and network components such as routers, switches, and firewalls.

The REST APIs would typically be used to facilitate key management and distribution between a KME and an SAE. These REST APIs are part of a higher-level Key Management System (KMS) that integrates with the QKD system.

The API is in the context of this document mainly used to manage the distribution of the keys and key IDs generated by the QKD devices.

In this QKD-enabled environment, these APIs would interact with the QKD system to ensure that the keys being managed and distributed are derived from quantum-safe processes. The KME would serve as the intermediary between the QKD system and the application layer, ensuring that keys are securely generated, distributed, and managed.

### Entities involved in the communication

- Key Management Entity (KME) is the entity that manages keys in a network (it is in cooperation with one or more other KMEs);

- Secure Application Entity (SAE) is the entity that requests one or more keys from a KME (e.g. a SAE is an MX Juniper router, an EX Juniper switch, etc.);

- QKD Entity (QKDE) is the entity providing key distribution functionality (e.g. a QKD Module - key manager implementing an underlying QKD protocol).

All SAEs and KMEs must possess an identifier that allows to uniquely identify them in the network.

### REST APIs used in key exchange

Below, an outline of the main REST APIs that would be used for key and KeyID exchange in such a system:

- Key Request API

- Endpoint: /api/v1/keys/request Method: POST Description: This API is used by the SAE to request a new key from the KME. The request may include parameters such as the desired key length, key type, and any specific requirements related to the application.

- Key Retrieval API

- Endpoint: /api/v1/keys/{key_id} Method: GET Description: This API allows the SAE to retrieve a specific key using the key_id provided during the key request or through other methods.

- Key Deletion API

- Endpoint: /api/v1/keys/{key_id} Method: DELETE Description: This API allows the SAE or KME to delete a key that is no longer needed. This may be used to securely dispose of a key once it has been used or is no longer valid.

- Key Status API

- Endpoint: /api/v1/keys/{key_id}/status Method: GET Description: This API allows the SAE to check the status of a specific key, such as whether it is active, expired, or revoked.

- Key Exchange API

- Endpoint: /api/v1/keys/exchange Method: POST Description: This API facilitates the exchange of keys between different entities (e.g., from one SAE to another) under the supervision of the KME. This is useful in scenarios where keys need to be shared securely between different endpoints.

- Key ID Management API

- Endpoint: /api/v1/keyids Method: POST, GET, DELETE Description: This API allows the creation, retrieval, and deletion of KeyIDs, which are used to reference specific keys within the KME system. KeyIDs are essential for tracking and managing the lifecycle of keys in the system. Tipically :

  - OPEN_CONNECT (in source, in destination, inout QOS, inout Key_stream_ID, out status)

  - GET_KEY (in Key_stream_ID, inout index, out Key_buffer, inout Metadata, out status)

  - CLOSE (in Key_stream_ID, out status)

- Audit Logs API

- Endpoint: /api/v1/logs/audit Method: GET Description: This API provides access to audit logs related to key management activities, including key creation, retrieval, exchange, and deletion. This is critical for maintaining security and compliance.

### QKD-MACsec key exchange automation

The python script implemented for QKD/MACsec integration is parametric and can be used for on-box (i.e. running as an event script inside a Juniper device, and scheduled using event/options in JUNOS), or off-box, on a remote server, and scheduled via Linux cron daemon.

Static CAK mode is implemented for links connecting switches or routers. Static CAK mode ensures security by frequently refreshing to a new random security key and by sharing only the security key between the two devices on the MACsec-secured point-to-point link.

When you enable MACsec using static CAK mode, two security keys are used to secure the link: 1. a connectivity association key (CAK) that secures control plane traffic and 2. a randomly-generated secure association key (SAK) that secures data plane traffic.

Both keys are regularly exchanged between both devices on each end of the p2p Ethernet link to ensure link security.

### Key exchange mechanism between 2 Juniper devices

The ETSI standards setting organization defined a REST API to exchange keys between a Key Management Server [KMS] (typically included in the QKD system) and a security application entity [SAE] such as Juniper MX.

The API itself is secured by TLS (issuing client certificates via an available PKI CA Root Certification Authority), however this is not Quantum-safe. For that reason, it is only allowed to be operated in a physically secure compartment. Such compartment can for example be a rack with locked doors or a locked room. In practice this means that the QKD system and the secure application device are in proximity to each other and do not exchange key material over the Internet.

Overall process view

In a typical scenario, each SAE must first identify itself to the local KME to start setting up a quantum secure MACsec L2 tunnel.

MX1 = SAE-A
MX2 = SAE-B
QKD1 = KME-A
QKD2 = KME-B

- The two SAEs need to elect an initiator and a responder. In the Juniper implementation this tie-breaking is part of the IKEv2 initialization process and is defined in the targets_dict (see the section 2.1.2.3 Configuration) from the ETSIA script. In the following we assume “SAE-A” is the initiator (master) role and “SAE-B” as responder (slave).

- Certificates are exchanged between SAEs (the Juniper router or switch) and the KMEs (the QKD server generating the quantum key material) for TLS communication exchange between them.

- The SAE indicates its preference to use QKD-keys to the MACsec peer (e.g.: MX1 being a primary device TLS queries the key and keyID pair from QKD1. MX1 will configure MACsec with the key fetched from QKD1).

- If the peer also supports the use of QKD-keys, the Key exchange process gets scheduled to fetch keys.

- SAE_A requests a key and key-ID from its local KME

- SAE_A receives a key and key-ID

- SAE_A communicates the key-ID to SAE_B: MX1 sets the keyID at the interface description. This information is available to MX2 either through LLDP (enabled on both devices) or via scp secure copy from remote QKD/KME device. It is assumed each QKD/KME shares the same key-id at anyone time. KeyID was previously carried over LLDP protocol, however keyID transport via OUI Ethernet type 0x88b7 maybe be supported in the future. A new methodology that can outperform LLDP in terms of key ID transport accross the non-quantum channel between SAE_A and SAE_B, is the SCP protocol, which is implemented in this v3.0.0.0 script. SCP guarantees encryption of the KEY-ID material and negligible delay/jitter if compared to LLDP timers. Also LLDP states updates are near-second, therefore SCP performances outlines LLDP in this case.

- SAE_B requests the key corresponding to the received key-ID

- The local KME delivers the key to SAE_B

- SAE_B confirms having received the key corresponding to the key-ID: MX2 has used the keyID received from MX1 to fetch the corresponding key from KME2 and configures MACsec accordingly.

- Now SAE_A and SAE_B are ready to use the shared key in their communication.

- After both sides are in possession of their shared keys, those are mixed with the existing SA key materials to get Quantum safe keys.

- When the MACsec keys are synced on both ends, a hitless rollover happens at regular intervals, to guarantee an automatic re-keying mechanism.

Although the QKD Key manager module is designed for Key exchange syncronization, both MX1 and MX2 SAE devices receive their keys asyncronously via a non-quantum channel. There is in fact a deterministic timeframe where the configured keys will be out of sync on both Juniper devices.

To guarantee MACsec tunnel running during that period, the existing MACsec session will continue using the preceeding key.

The output below, shows the Display MACsec Key Agreement (MKA) session information to see the number of MKAs that are in progress, connectivity association key (CAK) type, CAK status, and MKA packet count activity. See the security mka sessions summary for reference.

lab@SAE_A> show security mka sessions summary
Interface    Member-ID                  Type        Status        Tx        Rx        CAK Name
ge-0/0/1     1F51BB9778BCB24D17F4BE68   primary     live          34        3         ABCD1234ABCD5678ABCD1234ABCD56786C00260045EBAF1EBB0D41FBDD9FF375
ge-0/0/1     D0AC8A5F4D5A39B5DD9F649B   preceding   active        97        63        ABCD1234ABCD5678ABCD1234ABCD5678B4C9C6A1426AB71B1D53479F3D795B0D

### Script main functions

The initial monolithic Python script has been refactored into a modular structure with separate functions, to improve readability, maintainability, and reusability. The section below details each function and its use.

#### Main KME/SAE interaction

The main function in this script is the central component that coordinates the execution of different parts of the program. It handles setting up logging, defining target devices and their respective information (ex: interfaces, kme, connectivity-association etc.) forming a 121 tunnel for a number of channelled devices, and determining whether to execute the code on-box or off-box. It also manages threading to process multiple devices concurrently (used only on the off-box implementation).

#### Detailed View

- Global Variables:

  - global prof

    - Declares that prof is a global variable, which might be used for performance profiling or logging purposes.

- Parse Arguments:

  - args = get_args()

    - Calls the get_args function to parse command-line arguments and stores the result in args.

- Initialize Logging:

  - log = initialize_logging(args)

    - Initializes logging based on the parsed command-line arguments and assigns it to log.

- Define Target Devices:

  - targets_dict:

    - A dictionary that contains information about various target devices. Each device entry includes details like IP addresses, interfaces, and KME (Key Management Entity) information.

    - Example Entries:

      - CA_server: Configuration for the CA server with details such as IP address, path, and credentials.

      - mx001, mx002, mx003, mx004: Various devices with their respective IP addresses, interfaces, and KME information.

- Process Off-Box:

  - Checks if onbox is False (indicating an off-box approach):

    - Logs and prints that an off-box approach is being taken.

    - Thread Management:

      - Creates a list of master, slave and additional slave devices.

      - Divides the devices among threads based on the number of available threads (maxthreads).

      - Creates and starts threads to process these devices using req_thread.

      - Waits for all threads to complete using join.

- Process On-Box:

  - Checks if onbox is True (indicating an on-box approach):

    - Logs and prints that an on-box approach is being taken.

    - Device Processing:

      - Opens a device connection using the Device() context manager.

      - Checks and renews certificates if necessary.

      - Applies initial MACsec and event-driven configuration and processes the device.

- Error Handling:

  - Both in off-box and on-box approaches, exceptions are caught and logged if any errors occur during the device processing.

- Script Entry Point:

  - if __name__ == '__main__':

    - Ensures that the main function is called only when the script is executed directly, not when imported as a module.

  - main()

    - Executes the main function.

  - prof.close()

    - Closes the profiling or logging instance (assuming prof is a profiler or logger that needs to be closed).

The process function interacts with a JUNOS device to fetch keys from a Key Management Entity (KME) and update the MACsec CAK (Connectivity Association Key) configuration accordingly. Below is a detailed breakdown of what the function does:

#### Function Purpose

- Purpose: To fetch keys from a KME and apply or update the MACsec CAK configuration on a JUNOS device. This includes checking existing configurations, retrieving necessary information, and applying new configurations.

#### Parameters

- dev: The JUNOS device object used to interact with the device’s configuration.

- targets_dict: A dictionary that contains information about various target devices. Each device entry includes details like IP addresses, interfaces, and KME (Key Management Entity) information.

- log: A logging object used to record informational messages and errors.

#### Function Steps

- Define Configuration Filter:

  - Creates an XML filter to retrieve the MACsec configuration and interfaces from the JUNOS device.

- Fetch and Log Current Configuration:

  - Retrieves the current configuration using the defined filter.

  - Logs the configuration in a human-readable format.

- Parse MACsec Configuration:

  - Extracts the MACsec configuration from the retrieved XML.

  - Finds interfaces that have the qkd apply-macro.

- Handle Missing Interfaces:

  - Logs a critical error and exits if no interfaces with qkd apply-macro are found.

- Initialize Key Retrieval:

  - Creates a session for HTTPs requests.

  - Retrieves previously saved key-IDs, or if key-ID is not provided, sends a POST request to the KME API endpoint for encryption keys (enc_keys). The request includes additional slave SAE IDs if specified.

  - Initializes placeholders for new MACsec and interface configurations.

- Process Each Device:

  - Iterates through all the devices extracted from the targets_dict (in the case of off-box) or Iterates through interfaces that have the qkd apply-macro (in the case of on-box) and depending on the roles handle it respectively:

    - Master Device Handling:

      - Fetches keys from the KME if the local device is the master.

      - Updates the MACsec configuration with the received key.

    - Slave Device Handling:

      - Attempts to retrieve key-IDs from master, using SCP, if the local device is a slave.

      - Validates and fetches keys from the KME if necessary.

    - Saves the key IDs:

      - Get the Key_ID from the master device.

      - Key-ID are saved to a JSON file if present.

      - Retrieves the previous key IDs from a JSON file.

- Update MACsec Configuration:

  - Updates the connectivity association name and key information in the MACsec configuration XML.

  - Adds the updated configurations to the list of configurations to be committed.

- Commit Configuration:

  - Commits the updated configuration to the JUNOS device if there are changes.

  - Handles errors and logs success or failure.

- Save Key IDs:

  - Saves the new key IDs to a persistent JSON file.

- Logging and Completion:

  - Logs the success of the script execution.

### Multi threading function

The request_thread function designed to manage Juniper network devices, performing tasks such as renewing certificates and applying initial MACsec configurations, assuming an environments where multiple Juniper network devices need to be configured or updated simultaneously, reducing the time needed to manage large networks. Each instance of this function is intended to run in its own thread, allowing for parallel processing of multiple Juniper devices at once. It connects to each device via SSH, checks local and Root CA certificates, and applies configurations as needed. A configuration function checks the device’s MACsec initial configuration and applies any necessary changes, using the targets_dict dictionary which contains specific JunOS MX initial MACsec configuration parameters for each device. It’s also profiled for performance measurement, and logs its progress for any errors encountered, ensuring that each step can be tracked and debugged if necessary.

### Certificate life cycle management (LCM)

In case a RootCA isn’t provided, a generate_certificate function automates the process of generating a self-signed CA certificate and its associated private key. It sets various attributes like the subject name, certificate version, validity period, and extensions. The resulting certificate and key are then saved to specified file paths for later use in a Public Key Infrastructure (PKI). Note: this function can be currently used only on the off-box implementation of the script (PyOpenSSL not being part of the Junos python3 modules).

#### Generate client certificate function

It automates the process of generating a client certificate signed by the aforementioned (or any existing) Root CA certificate. It handles the generation of the client’s private key, the creation of the client certificate with necessary details and extensions, and then signs the certificate using the CA’s private key. Finally, it saves both the client certificate and the private key to the specified file paths.

#### Is certificate valid function

It checks whether a given X.509 certificate is valid for at least a specified number of days. This function helps ensure that a certificate is not about to expire and provides a way to proactively manage certificate validity.

#### Create SSH client function

It is used to establish an SSH (Secure Shell) connection to a remote device. It leverages the paramiko library, which is a Python implementation of the SSH protocol, to manage the connection. This function is essential for automating interactions with remote devices over SSH.

#### Get certificates function

It is used to obtain certificate files either locally from the device where it is running (e.g. a Juniper MX) or remotely from the device over SSH connection. It constructs file paths based on the environment and handles the retrieval process accordingly. If running on the device (onbox is true), it simply constructs and returns local file paths. If running externally (onbox is false), it connects to the device via SSH, downloads the certificate files, and then returns the local paths for these files.

#### Upload certificates function

It handles the process of uploading certificate files from a local machine to a remote device using SCP over SSH. It starts by creating an SSH client connection to the device, then uses SCP to transfer the certificate and key files to the device’s file system. If any issues arise during the upload process, they are caught and reported. This function is crucial for scenarios where certificates need to be deployed or updated on remote devices.

#### Fetch CA certificate function

It is used to connect to a remote CA server via SSH, download the CA certificate and key files, and save them locally. The function handles establishing the SSH connection, performing the SCP file transfer, and handling any errors that might occur during the process. It returns the local file paths where the downloaded certificate and key are stored. The local path where the certificates will be stored is determined based on whether the code is running on the device (onbox) or not.

* CERTS_DIR: Directory used if running on the device.
* OFFBOX_CERTS_DIR: Directory used if running off the device.

#### Renew certificates function

It is responsible for managing the lifecycle of certificates used by devices. It checks if the existing certificates are valid or if they are missing. If necessary, it fetches the CA certificate, generates new client certificates, and uploads the updated certificates to the device. This ensures that devices have up-to-date certificates for secure communication. If the onbox flag is False, indicating that the script is not running on the device, the function retrieves the current paths to the CA and client certificates and keys using the get_certificates function. This function provides the paths to these certificates that may need to be renewed.

#### Should check certs function

It determines whether it is time to check the validity of certificates based on the current date and time. The function determines if it’s time to perform a certificate validity check based on:

- Whether the current time is between midnight and 1 AM.
- Whether the current day of the month is a specific day (i.e., the 1st, 6th, 11th, 16th, or 21st).

This scheduling mechanism ensures that certificate validity checks occur at regular intervals during specific hours of the day, helping to maintain the validity and security of certificates without manual intervention.

### Logging and tracing

#### Initialize logging function

It sets up logging with various configurations:

* Defines a custom log level (NOTICE).
* Adds methods to the Logger class for this custom level.
* Configures logging based on user verbosity and other options (trace file, on-box logging).
* Sets up both console and file handlers, with different levels and formats as required.
* Provides feedback on successful initialization and returns the configured logger instance.

#### Get Args function

It is designed to define and parse command-line arguments for a Python script. It uses the argparse module to handle command-line arguments, allowing users to specify various options when running the script. Here’s a breakdown of the function:

### Initial configuration

#### Check and apply initial config function

It ensures that an initial MACsec (Media Access Control Security) configuration is applied to a network device if it hasn’t been already. Here’s a detailed explanation of how the function works:

#### Function Purpose

- Purpose: To check if the initial MACsec configuration is already applied to a device and, if not, to apply it.

#### Parameters

- dev: An object representing the network device, which provides methods for interacting with the device’s configuration.

- targets_dict: A dictionary containing target device details, including IP addresses, interfaces, and KME (Key Management Entity) details.

- log: A logging object used to log informational messages and errors.

#### Function Steps

- Extract Device and Configuration Details:

  - device_name: The hostname of the device from the dev object.

  - device_ip: The IP address of the device from targets_dict.

  - c_a: The connectivity association name used in the MACsec configuration.

  - interfaces: A list of interfaces on the device that will be configured.

  - kme_name: The name of the KME.

  - kme_ip: The IP address of the KME.

- Check Existing Configuration:

  - Uses the device’s RPC (Remote Procedure Call) to retrieve the current configuration related to the MACsec connectivity association.

  - Checks if the configuration is already applied by looking for the presence of a <name> element for a specific configuration filtered by security.macsec.connectivity-association[name=‘c_a’]. If found, logs a message and exits the function early.

- Define Initial MACsec Commands:

  - Basic Commands: Defines a set of commands for setting up the MACsec connectivity association, including cipher suite, security mode, and pre-shared keys.

  - Interface-Specific Commands: Extends the list of commands to configure each interface with MACsec settings, static host mappings, SCP, and event options.

- Apply Configuration:

  - Logging: Logs a message indicating the start of the configuration process.

  - Apply Commands:

    - Sets a timeout for the device interaction.

    - Uses a context manager (Config) to lock the configuration, load the commands, and commit the changes.

    - Logs success if the configuration is applied successfully, or logs an error if there is an issue.

- Cleanup and Delay:

  - Unlocks the configuration to allow further changes.

  - Introduces a 60-second sleep to ensure that the configuration is applied consistently across all devices.

### MACsec configuration script main functions

A list of interfaces are considered as input values to start the MACsec tunnel.

It’s recommended that CKN length be 64 hex digits (32 octets) irrespective of the cipher suite used (128 bits or 256 bits). The length of CKN is flexible although there is a requirement that it has to be between one to 32 octets long, which would mean two to 64 hex digits. An odd number of hex digits is not permitted.

We recommend using full-length CAKs. This means 32 hex digits (16 Octets, 128 bits) for 128-bit AES, and 64 hex digits (32 octets, 256 bits) for 256-bit AES. CAK length is extremely important for interoperability with third party MACsec devices because a smaller-than-required CAK could result in incompatible padding of zeroes by a third-party vendor when compared to Juniper.

The following CAK and CKN values are given as an example reference, to represnet the correct string format type&lenght when configured on a Junier device.

ckn_example = abcd1234abcd5678abcd1234abcd5678abcd1234abcd5678abcd1234abcd5678

cak_example = abcd1234abcd5678abcd1234abcd5678abcd1234abcd5678abcd1234abcd5678

The kme_a-qkd-remote-server-FQDN is the FQDN of the remote KME_A.

The following code, specifies protocols whose packets are not secured using Media Access Control Security (MACsec) when MACsec is enabled on a link using static connectivity association key (CAK) security mode. When this option is enabled in a connectivity association that is attached to an interface, MACsec is not enabled for all packets of the specified protocols that are sent and received on the link.

See the Juniper MACsec exclude-protocol reference documentation online.

# set security macsec connectivity-association CA_basic exclude-protocol lldp

set security macsec connectivity-association {c_a} cipher-suite gcm-aes-xpn-256  
set security macsec connectivity-association {c_a} security-mode static-cak  
set security macsec connectivity-association {c_a} pre-shared-key ckn <ckn_example> 
set security macsec connectivity-association {c_a} pre-shared-key cak <cak_example>

set security macsec interfaces {interface} apply-macro qkd kme-ca false
set security macsec interfaces {interface} apply-macro qkd kme-host {kme_a-qkd-remote-server-FQDN}
set security macsec interfaces {interface} apply-macro qkd kme-port 443
set security macsec interfaces {interface} connectivity-association {c_a}
set security macsec interfaces {interface} apply-macro qkd kme-keyid-check true
set system static-host-mapping {kme_name} inet {kme_ip}
set system static-host-mapping {device_name} inet {device_ip}

#### Event-options

# devices must be in sync with NTP
set event-options generate-event every10mins time-interval 600 <start-time start-time>
set event-options policy qkd events every10mins
set event-options policy qkd then event-script onbox.py
set event-options event-script file onbox.py python-script-user remote
set event-options traceoptions file script.log
set event-options traceoptions file size 10m

#### Other necessary config

# set system host-name SAE_A
set system static-host-mapping [kme_a-qkd-remote-server-FQDN] inet [qkd-management-IP-addr]
set system static-host-mapping [DEVICE_A] inet [DEVICE_A_IP]

#### Description of the macros

| apply-macro | Use |
| --- | --- |
| kme-ca (true, false) | When set to true, uses HTTPS(SSL/TLS) to verify connection to KME |
| kme-host | Hostname or IP address of the KME connected to the SAE. Make sure you add a static host-name Junos config if you use hostname |
| kme-method (get, post) | Specifies the method used to fetch key with keyID in secondary device |
| kme-port | Port used to talk to KME. Should be set to 443, unless you need to experiment |
| kme-keyid-check (true, false) | If set to false, get/post call to KME will be made even if KeyId is stale. This macro is ignored for Primary SAE |

### Key Management functions

#### Fetch KME key function

It is responsible for fetching keys from a Key Management Entity (KME) using HTTP requests.

- Function Parameters:

  - session: An active requests.Session object used for making HTTP requests.

  - local_name: The local identifier used to derive paths for client certificate and key.

  - log: A logger instance for logging messages and errors.

  - remote_mnmgt_add: The remote management address to be used in the KME URL.

  - kme_url: The base URL of the KME.

  - key_id: An optional parameter specifying the ID of the key to fetch.

  - additional_slave_SAE_IDs: an optional parameter used for specifying two or more slave SAEs to share identical keys.

- Certificate and Key Paths:

  - The function constructs paths for the client certificate (client_crt) and client key (client_key) based on the local_name and the directory CERTS_DIR.

  - CLIENT_CERT is a tuple containing the paths to these certificate and key files.

- Fetching Keys:

  - If key_id is provided, the function constructs a URL to request decryption keys (dec_keys) from the KME, appending the key_id to the base URL.

  - If key_id is not provided, the function constructs a URL to request encryption keys (enc_keys).

  - The constructed URL includes the remote_mnmgt_add for the remote management address (peer SAE).

  - The requests.Session.get() method is used to perform the HTTP GET request. The request includes the verify parameter set to CA_CERT to specify the CA certificate for SSL verification and cert set to CLIENT_CERT to provide client-side SSL credentials.

- Error Handling:

  - If an exception occurs during the request (such as connection errors or invalid responses), the function logs an error message and returns None.

- Return Value:

  - If the request is successful, the function returns the JSON response from the KME, which contains the requested keys.

#### Get previous key ids function

It performs the following tasks:

- Attempts to open and read a JSON file to retrieve previous key IDs.

- Handles scenarios where the file is missing or contains invalid data by logging appropriate messages.

- Returns an empty dictionary if the file is not found or if it contains no valid key IDs.

This approach ensures that the function is resilient to common issues like file absence or data corruption, while providing informative logs to help diagnose such issues.

#### Save key ids function

It is designed to save a dictionary of key IDs to a JSON file.

# Script python3 functions deep dive

## Purpose of the python script

This script automates the full lifecycle required to configure QKD-backed MACsec on network devices. It handles:

- Secure SSH connectivity

- Certificate generation, renewal, and upload

- Retrieval of QKD keys from a KME

- Distribution of key IDs between devices

- Applying initial and periodic MACsec configuration

- Multithreaded off-box management of many devices

- Profiling and logging for debugging and traceability

It supports two execution modes:

- Offbox mode — Run from a management host and configure multiple devices in parallel.

- Onbox mode — Run directly on a device, performing the full workflow locally.

The script is separated into logical components (threads, certificates, SSH, KME interactions, MACsec config, etc.), with each function contributing toward completing the workflow.

Below is a description of every function, grouped by subsystem, showing how they work together.

## Threading / Parallel Execution

#### background

- background(func) A decorator that runs the wrapped function asynchronously in a background thread.

- Creates a Thread

- Starts it as a daemon

- Tracks it in the global threads list

Used primarily to parallelize device operations in offbox mode.

#### req_thread

- req_thread(tnum, reqs, targets_dict, log) Worker function executed in background threads.

- Iterates over a subset (reqs) of devices assigned to this thread.

- For each device:

  - Creates a connection (Device)

  - Applies initial config

  - Fetches and pushes keys

  - Applies final MACsec configuration

Contributes to scaling the script for multi-device deployments.

## Certificate Management

#### generate_ca_certificate

- generate_ca_certificate(ca_cert_path, ca_key_path, ca_subject)

Creates a self-signed CA certificate and private key, used for signing client certificates.

Used when the CA certificate is missing or expired.

#### generate_client_certificate

- generate_client_certificate(client_cert_path, client_key_path, ca_cert_path, ca_key_path, client_subject)

Generates a client certificate/key pair signed by the CA.

Uploaded later to the device for secure KME communication.

#### is_certificate_valid

- is_certificate_valid(cert_path, min_valid_days=10)

Checks if a certificate exists and whether it is still valid for at least min_valid_days days.

Used before renewing certificates.

#### get_certificates

- get_certificates(dev, log, targets_dict)

Downloads the device’s existing client certificate and key over SSH.

Used to validate and determine whether renewal is required.

#### upload_certificates

- upload_certificates(dev, cert_path, key_path, log, targets_dict)

Uploads new or renewed certificates back onto the device via SCP.

#### fetch_ca_certificate

- fetch_ca_certificate(targets_dict)

Retrieves (or generates) the CA certificate on the management host.

May be used to distribute a trusted CA to devices.

#### renew_certificates

- renew_certificates(dev, log, targets_dict)

Central certificate workflow on a device:

- Check existing cert validity

- If invalid or missing:

  - Generate new client cert

  - Upload it to the device

- Log success/failure

Used within the main processing flow.

#### should_check_certs

Tells the script if certificate validation should be performed. Triggered via command-line flags or environment conditions.

## Logging / CLI

#### initialize_logging

- initialize_logging(args) Configures logging:

- Verbosity (-v)

- Trace mode (--trace)

- Logging to file or console

Essential for debugging distributed device operations.

#### get_args

Defines command-line parameters for:

- Thread count

- Verbose output

- Trace logs

Used by main() to interpret user input.

## Key ID Tracking

#### get_previous_key_ids

- get_previous_key_ids(log, name)

Reads previously stored key IDs from the device’s JSON file.

Used to determine if a new key has been fetched and prevent unnecessary work.

#### save_key_ids

- save_key_ids(key_dict, local_name)

Writes the key IDs into a JSON file for persistent tracking.

## KME Key Retrieval

#### fetch_kme_key

- fetch_kme_key(session, local_name, log, remote_mnmgt_add, kme_host, key_id, additional_slave_SAE_IDs=None)

Contacts the KME and retrieves a fresh QKD key for the specified device.

- Negotiates session keys

- Stores the returned key material locally

- Updates key IDs for master/slave/additional devices

Core function for QKD key retrieval.

## Device MACsec Configuration

#### check_and_apply_initial_config

- check_and_apply_initial_config(dev, targets_dict, log)

Runs initial device configuration:

- Basic MACsec setup

- CA/client certificate preparation

- Upload of trust anchors

- Validation checks

Executed once per device unless the device already has MACsec config.

#### get_key_id_from_master

- get_key_id_from_master(dev, log, targets_dict)

Retrieves the master’s Key ID:

- Offbox: Reads from master’s JSON file

- Onbox: SCP fetches key ID file

Used so slaves can request the correct KME key.

#### process

- process(dev, targets_dict, log)

The central device workflow:

- Renew certificates (if needed)

- Fetch key ID from master (if slave)

- Retrieve new KME key

- Apply MACsec key + policy configuration

- Save the new key ID locally

This is what each thread or onbox execution ultimately performs.

## Main Program Flow

#### main

Entry point of the script.

Responsible for:

- Parsing CLI arguments

- Initializing logging

- Loading configuration (targets_dict)

- Deciding Onbox vs Offbox execution

- Offbox:

  - Split device list into thread groups

  - Call req_thread() for each

  - Wait for all threads to finish

- Onbox:

  - Run full process() directly and sequentially

This function connects everything into a complete automated workflow.

## End-to-End Workflow Summary

The entire script implements this high-level pipeline:

Parse Args → Init Logging → Load Targets → (Offbox: Threading)  
     ↓  
Connect to Device  
     ↓  
Certificate Handling (validate, renew, upload)  
     ↓  
Retrieve Key ID  
     ↓  
Fetch QKD Key from KME  
     ↓  
Apply MACsec Configuration  
     ↓  
Save Key IDs → Done

This structure ensures automated, scalable, secure MACsec provisioning with QKD key material across many devices.

## Environment configuration block.

- Defines a base directory (CUR_DIR) path and certificate path

- Builds all file paths relative to that directory.

- Sets up database connection configuration settings.

- Specifies TLS certificates.

- Defines API URLs.

- Creates logging locations and profiling locations.

CUR_DIR means “current working directory for this application” It is simply the base folder from which the program reads/writes files like: * certs * logs * JSON output * profiling data

OFFBOX_CERTS_DIR stores certificates: * TLS certificate * TLS private key * Root CA

DATABASE_PORT = '10000'
DATABASE_HOST = '9.173.9.102'
DATABASE_USER = 'db_user'
DATABASE_PASSWORD = 'db_password'
DATABASE_URL = f'postgres://{DATABASE_USER}:{DATABASE_PASSWORD}@{DATABASE_HOST}:{DATABASE_PORT}/key_store'

These define how the ETSI-014 reference implementation connects to PostgreSQL. Final URL produced:

postgres://db_user:db_password@9.173.9.102:10000/key_store

CERTS_DIR = f'{CUR_DIR}/certs/'
Same as OFFBOX_CERTS_DIR, used to store:
CA_CERT = f'{CERTS_DIR}/root.crt'

This is the path to the CA certificate.

ETSI_014_REF_IMPL_TLS_CERT = f'{OFFBOX_CERTS_DIR}/kme_001.crt'
ETSI_014_REF_IMPL_TLS_PRIVATE_KEY = f'{OFFBOX_CERTS_DIR}/kme_001.key'
ETSI_014_REF_IMPL_TLS_ROOT_CRT = f'{OFFBOX_CERTS_DIR}/root.crt'

These are TLS files for a Key Management Entity (KME) implementation.

## Managing multithreading

The threads[] list will hold every thread that the decorator creates.This list will hold every thread the program starts. It allows the program to keep track of all the work happening in parallel. Useful if you later want to: * wait for them * check their status * debug how many threads are running

The decorator starts that function in a separate background thread. The thread is set as a daemon thread, which means it does not block the program from exiting. The thread starts immediately. The thread object is added to the threads list for tracking. The original function runs asynchronously (in parallel), without waiting for it to finish. In simple terms: Putting @background above a function makes it run in the background.

### Threading code

threads = []

This list will hold every thread the program starts. It allows the program to keep track of all the work happening in parallel.

The decorator runs functions in the background

def background(func):
    def bg_func(*args, **kwargs):
        t = Thread(target=func, args=args, kwargs=kwargs)
        t.setDaemon(True)
        t.start()
        threads.append(t)
    return bg_func

This is a decorator. Decorators wrap a function and change how it behaves.

### Decorator function

- When you call a decorated function, it starts that function in a separate background thread.

- The thread is set as a daemon thread, which means it does not block the program from exiting.

- The thread starts immediately.

- The thread object is added to the threads list for tracking.

- The original function runs asynchronously (in parallel), without waiting for it to finish.

In simple terms:

Putting @background above a function makes it run in the background.

A threaded function that processes devices

@background
def req_thread(tnum, reqs, targets_dict, log):

By adding @background, this function now runs in its own thread whenever it’s called.

req_thread() # Explained

This function is designed to run on one thread and process a list of devices.

Here’s what it does step-by-step:

#### Start profiling for the whole thread

prof.start("Thread-{0}".format(tnum))

This logs performance information for the thread.

#### Loop through each device in the list

for device in reqs:
    print(device)

It prints each device name as it works on it.

#### Connect to the network device

with Device(host=targets_dict[device]['ip'],
            user=targets_dict["secrets"]["username"],
            password=targets_dict["secrets"]["password"],
            port=22) as dev:

For each device:

- It opens an SSH session to the Juniper device.

- The connection automatically closes when done.

#### Optionally renew certificates

if should_check_certs():
    renew_certificates(dev, log, targets_dict=targets_dict)

If the program decides certificates need checking, it:

- runs certificate renewal

- logs the activity

- profiles how long it took

#### Apply initial configuration

check_and_apply_initial_config(dev, targets_dict, log)

This step ensures that the device has the correct initial configuration before performing further tasks.

Profiling is used around this step to measure performance.

#### Run the main processing logic

process(dev, targets_dict, log)

This is the main work done for each device. It could be anything: pushing config, collecting data, etc.

Again, profiling measures how long it takes.

#### Stop profiling at the end of the thread

prof.stop("Thread-{0}".format(tnum))

This finalizes all timing measurements for the thread.

### Summary

Done: The @background decorator makes any function run in a background thread.

Done: req_thread() is a function that:

- receives a list of network devices

- connects to each device

- optionally renews certificates

- applies initial configuration

- performs the main processing logic

- logs and profiles everything

- does each device in order

- but runs the entire function in its own thread

Done: Multiple req_thread() calls allow the program to work on many devices in parallel, each handled by a separate thread.

# All python functions explained

## generate_ca_certificate

This function creates a self-signed Certificate Authority (CA) certificate and its private key, and saves them as files.

- Function Signature

def generate_ca_certificate(ca_cert_path, ca_key_path, ca_subject):

Parameters:

- ca_cert_path → file path where the CA certificate will be saved (PEM format)

- ca_key_path → file path where the CA private key will be saved (PEM format)

- ca_subject → the “common name” (CN) for the CA (e.g., "My Root CA")

Returns:

- Tuple (ca_cert_path, ca_key_path) → paths of the saved certificate and key files

- Generate the CA private key

ca_key = crypto.PKey()
ca_key.generate_key(crypto.TYPE_RSA, 2048)

- Creates a 2048-bit RSA key, which will be used to sign the CA certificate.

- This key is the secret that proves the CA’s identity.

- Create the CA certificate object

ca_cert = crypto.X509()
ca_cert.set_version(2)
ca_cert.set_serial_number(int(uuid.uuid4()))
ca_cert.get_subject().CN = ca_subject
ca_cert.set_issuer(ca_cert.get_subject())
ca_cert.set_pubkey(ca_key)

- X509() → creates a new certificate object.

- set_version(2) → X.509 version 3 certificate (0-based counting in OpenSSL: version 2 = v3).

- set_serial_number(int(uuid.uuid4())) → unique serial number using a UUID.

- get_subject().CN = ca_subject → sets the Common Name (CN) field of the certificate.

- set_issuer(ca_cert.get_subject()) → issuer is the same as subject → self-signed.

- set_pubkey(ca_key) → associate the public key with the certificate.

- Set validity period

ca_cert.gmtime_adj_notBefore(0)
ca_cert.gmtime_adj_notAfter(5 * 365 * 24 * 60 * 60)  # 5 years

- Certificate is valid from now (notBefore) until 5 years (notAfter).

- Time is expressed in seconds.

- Add X.509 extensions

ca_cert.add_extensions([
    crypto.X509Extension(b"subjectKeyIdentifier", False, b"hash", subject=ca_cert),
    crypto.X509Extension(b"basicConstraints", False, b"CA:TRUE"),
])

- subjectKeyIdentifier → allows others to identify this certificate’s public key.

- basicConstraints → marks this certificate as a Certificate Authority (CA:TRUE).

- Some other extensions are commented out (authorityKeyIdentifier, keyUsage), but could be added later.

- Sign the certificate

ca_cert.sign(ca_key, 'sha256')

- Uses the CA private key to sign the certificate with SHA-256.

- This makes it a valid self-signed CA certificate.

- Save the certificate and key to files

with open(ca_cert_path, "wb") as f:
    f.write(crypto.dump_certificate(crypto.FILETYPE_PEM, ca_cert))

with open(ca_key_path, "wb") as f:
    f.write(crypto.dump_privatekey(crypto.FILETYPE_PEM, ca_key))

- Writes the certificate and private key in PEM format.

- These files can be used later to sign server or client certificates.

- Return the paths

return ca_cert_path, ca_key_path

- Returns the file paths for reference, so the caller knows where they were saved.

- Summary

- Create a new RSA key (private key for the CA).

- Create a new X.509 certificate object.

- Fill in the certificate info (subject, issuer, serial number, public key).

- Set how long the certificate is valid (5 years).

- Add CA-specific extensions (marks it as a certificate authority).

- Sign the certificate using the CA private key (self-signed).

- Save both certificate and private key to files.

- Return the file paths.

Here’s a plain-language explanation in Markdown of your generate_client_certificate function, step by step.

## generate_client_certificate

This function creates a client certificate that is signed by an existing CA (Certificate Authority). It also generates a private key for the client and saves both files.

- Function Signature

def generate_client_certificate(client_cert_path, client_key_path, ca_cert_path, ca_key_path, client_subject):

Parameters:

- client_cert_path → where the client certificate will be saved

- client_key_path → where the client private key will be saved

- ca_cert_path → path to the CA certificate (used to sign the client cert)

- ca_key_path → path to the CA private key

- client_subject → the “Common Name (CN)” for the client certificate

Returns:

- Tuple (client_cert_path, client_key_path) → paths of saved certificate and key files

- Load the CA certificate and key

ca_key = crypto.load_privatekey(crypto.FILETYPE_PEM, open(ca_key_path, 'rb').read())
ca_cert = crypto.load_certificate(crypto.FILETYPE_PEM, open(ca_cert_path, 'rb').read())

- Reads the CA private key and CA certificate from disk.

- These are used to sign the client certificate, proving trust.

- Generate a new client key

client_key = crypto.PKey()
client_key.generate_key(crypto.TYPE_RSA, 2048)

- Creates a 2048-bit RSA key for the client.

- This key is secret and will be stored in a file for the client to use.

- Create the client certificate object

client_cert = crypto.X509()
client_cert.set_version(2)
client_cert.set_serial_number(int(uuid.uuid4()))
client_cert.get_subject().CN = client_subject
client_cert.set_issuer(ca_cert.get_subject())
client_cert.set_pubkey(client_key)
client_cert.gmtime_adj_notBefore(0)
client_cert.gmtime_adj_notAfter(10*365*24*60*60)

- X509() → create a new certificate object

- set_version(2) → X.509 version 3 certificate

- set_serial_number(int(uuid.uuid4())) → unique serial number

- get_subject().CN = client_subject → sets the client’s name

- set_issuer(ca_cert.get_subject()) → issuer is the CA

- set_pubkey(client_key) → attach client’s public key

- gmtime_adj_notBefore(0) → valid from now

- gmtime_adj_notAfter(...) → valid for 10 years

- Add X.509 extensions

client_cert.add_extensions([
    crypto.X509Extension(b"basicConstraints", False, b"CA:FALSE"),
    crypto.X509Extension(b"authorityKeyIdentifier", False, b"keyid:always", issuer=ca_cert),
    crypto.X509Extension(b"keyUsage", False, b"Digital Signature, Non Repudiation, Key Encipherment, Data Encipherment"),
])

- basicConstraints → marks this certificate as not a CA

- authorityKeyIdentifier → links this cert to the CA that signed it

- keyUsage → defines what the certificate can be used for (signing, encryption, etc.)

- Other extensions like subjectKeyIdentifier and extendedKeyUsage are commented out but could be added for client authentication.

- Sign the client certificate

client_cert.sign(ca_key, 'sha256')

- Uses the CA private key to sign the client certificate.

- This makes it trusted by anyone who trusts the CA.

- Save the client certificate and key

with open(client_cert_path, "wb+") as f:
    f.write(crypto.dump_certificate(crypto.FILETYPE_PEM, client_cert))

with open(client_key_path, "wb+") as f:
    f.write(crypto.dump_privatekey(crypto.FILETYPE_PEM, client_key))

- Saves both files in PEM format.

- These files can be used by the client for authentication.

- Return the paths

return client_cert_path, client_key_path

- Returns the file paths so the caller knows where the certificate and key were saved.

- Summary

- Load the CA certificate and private key (the signer).

- Generate a new RSA key for the client.

- Create a new certificate object for the client.

- Fill in client info (name, issuer, public key, validity).

- Add extensions (not a CA, key usage, link to CA).

- Sign the certificate with the CA’s key.

- Save the certificate and private key to files.

- Return the file paths.

## is_certificate_valid

This function checks whether a certificate file is still valid for at least a given number of days.

- Function Signature

def is_certificate_valid(cert_path, min_valid_days=10):

Parameters:

- cert_path → path to the certificate file (PEM format)

- min_valid_days → the minimum number of days the certificate should still be valid (default is 10)

Returns:

- True → certificate is valid for at least min_valid_days

- False → certificate expires sooner than min_valid_days

- Read the certificate file

with open(cert_path, 'rb') as cert_file:
    cert_data = cert_file.read()

- Opens the certificate file in binary mode (rb)

- Reads the contents into cert_data

- Load the certificate

cert = crypto.load_certificate(crypto.FILETYPE_PEM, cert_data)

- Converts the PEM data into an OpenSSL X.509 certificate object

- This allows access to the certificate fields like notAfter (expiration date)

- Parse the expiration date

not_after = datetime.datetime.strptime(cert.get_notAfter().decode('ascii'), '%Y%m%d%H%M%SZ')

- cert.get_notAfter() → returns the expiration date in ASN.1 format (bytes like b'20251127120000Z')

- .decode('ascii') → convert bytes to string

- datetime.strptime(..., '%Y%m%d%H%M%SZ') → convert string to a Python datetime object

Now not_after is the exact expiration date and time of the certificate.

- Calculate remaining validity

remaining_days = (not_after - datetime.datetime.now()).days

- Subtracts current date/time from the expiration date

- Gets the difference in days

- This tells us how many days are left until the certificate expires

- Check against minimum required days

return remaining_days >= min_valid_days

- Returns True if the certificate is valid for at least min_valid_days

- Returns False if it will expire sooner

- Summary

- Open the certificate file.

- Load it into a usable certificate object.

- Read its expiration date.

- Calculate how many days remain until it expires.

- Return True if it’s valid for at least the minimum days, otherwise False.

This function is useful for:

- Automated certificate renewal checks

- Preventing expired certificate errors in TLS/SSL connections

- Monitoring certificate health in network devices

## createSSHClient

This version of the function is designed to safely connect to a remote device over SSH with automatic retries and timeouts. It’s more robust than the previous version.

- Function signature

def createSSHClient(device, username, password, port=22, retries=3, delay=5, timeout=10):

- device → IP or hostname of the device

- username → SSH username

- password → SSH password

- port → SSH port, default 22

- retries → how many times to try connecting if it fails

- delay → seconds to wait between retries

- timeout → maximum time (seconds) to wait for SSH connection

- Retry loop

for attempt in range(1, retries + 1):

- Tries connecting multiple times if the first attempt fails.

- attempt counts the current try (1, 2, 3, …).

- Create and configure SSH client

client = paramiko.SSHClient()
client.set_missing_host_key_policy(paramiko.AutoAddPolicy())

- SSHClient() → creates a new SSH client object.

- set_missing_host_key_policy(AutoAddPolicy()) → automatically trusts unknown hosts instead of failing.

- Connect to the device

client.connect(
    hostname=device,
    port=port,
    username=username,
    password=password,
    timeout=timeout,
    banner_timeout=timeout,
    auth_timeout=timeout
)

- Attempts the actual SSH connection.

- Uses timeouts for connection, banner, and authentication to prevent hanging.

- Success message

print(f"Connected to {device} on attempt {attempt}")
return client

- If connection succeeds, prints a confirmation and returns the connected SSH client.

- Handle connection errors

except (SSHException, AuthenticationException, TimeoutError) as e:
    print(f"Attempt {attempt} failed to connect to {device}: {e}")

- Catches SSH errors, authentication failures, and timeout errors.

- Prints the error for visibility.

- Retry or fail

if attempt < retries:
    print(f"Retrying in {delay} seconds...")
    time.sleep(delay)
else:
    print(f"Failed to connect to {device} after {retries} attempts.")
    return None

- If there are retries left, waits for delay seconds and tries again.

- If this was the last attempt, prints a failure message and returns None.

- Summary

- Try to connect to the device via SSH.

- If it fails, retry up to retries times, waiting delay seconds each time.

- If connection succeeds, return the SSH client object.

- If all attempts fail, return None.

- Uses timeouts to prevent the program from hanging indefinitely.

- Automatically accepts unknown host keys.

This version is safe for looping over many devices because failed connections don’t crash the program, and successful connections can still be used for commands.

## get_certificates

This function retrieves TLS/SSL certificates for a given device, either from local files (on the device itself) or via SCP from a remote device.

- Function Signature

def get_certificates(dev, log, targets_dict):

Parameters:

- dev → a device object (likely from jnpr.junos.Device)

- log → logging object (for writing logs, not used here in the snippet)

- targets_dict → dictionary containing device info and credentials

Returns:

- ca_cert_path → path to the CA certificate

- ca_key_path → path to the CA private key

- client_cert_path → path to the client certificate

- client_key_path → path to the client private key

- Determine if running on the device (onbox)

if onbox:
    local_name = dev.facts['hostname']
    ca_cert_path = os.path.join(CERTS_DIR, 'client-root-ca.crt')
    ca_key_path = os.path.join(CERTS_DIR, 'client-root-ca.key')
    client_cert_path = os.path.join(CERTS_DIR, f'{local_name}.crt')
    client_key_path = os.path.join(CERTS_DIR, f'{local_name}.key')

- If the code is running directly on the device:

  - Use the device’s hostname as local_name.

  - Certificates are expected to be in CERTS_DIR.

  - Build paths for the CA certificate, CA key, client certificate, and client key.

- If running off-device (offbox)

else:
    local_name = dev.facts['hostname'].split('-re')[0]
    client = createSSHClient(local_name, username=targets_dict["secrets"]["username"], password=targets_dict["secrets"]["password"], port=22)

- If the code runs elsewhere:

  - Derive local_name from the hostname (remove -re suffix).

  - Create an SSH connection to the remote device using credentials in targets_dict.

- Copy certificates via SCP

with SCPClient(client.get_transport()) as scp:
    scp.get(remote_path=CERTS_DIR, local_path=OFFBOX_CERTS_DIR, recursive=True)

- Uses SCP (Secure Copy) to download all files from the device’s CERTS_DIR to local OFFBOX_CERTS_DIR.

- recursive=True ensures directories are copied fully.

- Build certificate paths

ca_cert_path = os.path.join(OFFBOX_CERTS_DIR, 'account-1286-server-ca-qukaydee-com.crt')
ca_key_path = os.path.join(OFFBOX_CERTS_DIR, 'account-1286-server-ca-qukaydee-com.crt')
client_cert_path = os.path.join(OFFBOX_CERTS_DIR, f'{local_name}.crt')
client_key_path = os.path.join(OFFBOX_CERTS_DIR, f'{local_name}.key')

- Sets paths to CA certificate and key (for testing they have the same file).

- Sets paths for client certificate and key using local_name.

- Handle SCP errors

except SCPException as e:
    print(f'SCP get exception error: {e}')

- Prints an error if SCP fails, e.g., network problem or missing files.

- Return certificate paths

return ca_cert_path, ca_key_path, client_cert_path, client_key_path

- Returns all four paths for use elsewhere, e.g., generating client certificates, validation, or TLS connections.

- Summary

- Determine if the script is running directly on the device (onbox) or elsewhere.

- If onbox, certificates are read locally.

- If offbox:

  - Connect via SSH to the device

  - Copy certificate files with SCP to a local folder

  - Build paths to CA certificate, CA key, client certificate, and client key

- Return the file paths for use by other functions.

- Errors during SCP are caught and printed.

This function is useful for centralizing certificate management across multiple network devices, ensuring both on-device and off-device workflows are supported.

## upload_certificates

This function uploads certificates to a remote device using SSH and SCP.

- Function Signature

def upload_certificates(dev, cert_path, key_path, log, targets_dict):

Parameters:

- dev → device object (likely from jnpr.junos.Device)

- cert_path → local path to the certificate file

- key_path → local path to the private key file

- log → logging object (not used in this snippet)

- targets_dict → dictionary containing device info and credentials

Returns:

- Nothing; it uploads files to the remote device.

- Determine the device IP

local_name = dev.facts['hostname'].split('-re')[0]
# TODO: line below used for testing
local_name = targets_dict[local_name]['ip']

- Uses the device hostname to look up the IP address from targets_dict.

- The split('-re')[0] removes any -re suffix from hostnames (device naming convention).

- For testing, it directly assigns the IP from targets_dict.

- Create an SSH connection

client = createSSHClient(local_name, username=targets_dict["secrets"]["username"], password=targets_dict["secrets"]["password"], port=22)

- Calls your improved createSSHClient function to establish a secure SSH session to the device.

- Upload files via SCP

with SCPClient(client.get_transport()) as scp:
    cert_files = [cert_path, key_path]
    scp.put(files=cert_files, remote_path=CERTS_DIR)

- Opens an SCP session over the SSH connection.

- Defines cert_files → a list of the certificate and key files to upload.

- Uploads the files to the device’s CERTS_DIR directory.

- Handle errors

except SCPException as e:
    print(f'SCP put exception error: {e}')

- Catches SCP exceptions, e.g., network errors, permissions issues.

- Prints a message instead of crashing the program.

- Summary

- Determine the IP of the device from its hostname.

- Connect to the device via SSH.

- Upload the certificate and private key files to the device using SCP.

- Catch and report any errors.

Note: Notes / Suggestions

- You could add return values or logging to indicate success/failure.

- Consider checking if the files exist locally before attempting upload.

- Using the retries and timeout from your SSH client function can make uploads more robust.

## fetch_ca_certificate

This function retrieves the CA (Certificate Authority) certificate and key from a remote CA server.

- Function Signature

def fetch_ca_certificate(targets_dict):

Parameters:

- targets_dict → dictionary containing information about the CA server and credentials

Returns:

- ca_cert_path → local path to the downloaded CA certificate

- ca_key_path → local path to the downloaded CA private key

- Get CA server details

ca_server_ip = targets_dict["CA_server"]["ca_server_ip"]
ca_user = targets_dict["CA_server"]["ca_user"]
ca_pass = targets_dict["CA_server"]["ca_pass"]
ca_path = targets_dict["CA_server"]["ca_path"]
ca_cert_name = targets_dict["CA_server"]["ca_cert_name"]
ca_key_name = targets_dict["CA_server"]["ca_key_name"]

- Reads IP, username, password, remote path, certificate name, and key name for the CA server from targets_dict.

- These are used to connect to the server and locate the files.

- Create an SSH connection to the CA server

client = createSSHClient(ca_server_ip, username=ca_user, password=ca_pass, port=22)

- Uses the createSSHClient function to establish a secure SSH connection.

- This allows the script to access files on the remote CA server.

- Determine local download path

LOCAL_PATH = CERTS_DIR if onbox else OFFBOX_CERTS_DIR

- If running on the device (onbox), save certificates to CERTS_DIR.

- If running off-device, save to OFFBOX_CERTS_DIR.

- Copy CA certificate and key via SCP

with SCPClient(client.get_transport()) as scp:
    scp.get(remote_path=ca_path, local_path=LOCAL_PATH, recursive=True)
    ca_cert_path = os.path.join(LOCAL_PATH, ca_cert_name)
    ca_key_path = os.path.join(LOCAL_PATH, ca_key_name)

- Opens an SCP session over SSH.

- Downloads all files in ca_path from the CA server to the local folder.

- Constructs local paths for the certificate and private key.

- Handle errors

except SCPException as e:
    print(f'SCP exception error: {e}')

- Catches errors like network failure, permissions issues, or missing files.

- Prints an error message instead of crashing.

- Summary

- Read CA server connection details from targets_dict.

- Connect to the CA server via SSH.

- Determine where to save the downloaded files locally.

- Use SCP to download the CA certificate and key from the server.

- Return the paths of the local CA certificate and key.

- If SCP fails, print an error message.

Note: Notes / Suggestions

- You could add checks to verify that the files actually exist after download.

- Using your retry-enabled SSH client here would make it more robust.

- Optionally, log successful downloads for auditing.

## renew_certificates

This function ensures that the CA and client certificates are valid on a device. It renews or generates certificates if they are missing or about to expire, and uploads them to the device if needed.

- Function Signature

def renew_certificates(dev, log, targets_dict):

Parameters:

- dev → device object (from jnpr.junos.Device)

- log → logging object for info messages

- targets_dict → dictionary with device info, CA server settings, and credentials

Returns:

- Nothing; performs actions (generates/renews/upload certificates) on the device.

- Get current certificate paths

if not onbox:
    ca_cert_path, ca_key_path, client_cert_path, client_key_path = get_certificates(dev, log, targets_dict=targets_dict)

- If running off-device, use get_certificates to download certificate paths from the device.

- If running on-device (onbox), paths are assumed to be local.

- Check if renewal is needed

if not os.path.isfile(client_cert_path) and not os.path.isfile(ca_cert_path):
    renew = True
elif not is_certificate_valid(client_cert_path):
    renew = True
else:
    renew = False

- Renew if:

  - Certificates don’t exist yet, or

  - The client certificate is about to expire (is_certificate_valid returns False)

- Otherwise, no renewal is needed.

- Renew or generate certificates

if renew:
    if targets_dict["CA_server"]["CA_cert"]["fetch"]:
        ca_cert_path, ca_key_path = fetch_ca_certificate(targets_dict)
    elif targets_dict["CA_server"]["CA_cert"]["generate"]:
        ca_cert_path, ca_key_path = generate_ca_certificate(ca_cert_path, ca_key_path, 'Juniper CA')
    else:
        log.info('The CA certificate was manually generated and uploaded')

- If renewal is needed:

  - Fetch CA certificate from a remote CA server if fetch is True.

  - Generate a new CA certificate locally if generate is True.

  - Otherwise, assume the CA certificate was manually uploaded.

- Generate a client certificate

generate_client_certificate(client_cert_path, client_key_path, ca_cert_path, ca_key_path, 'client')

- Creates a client certificate signed by the CA.

- Saves it to client_cert_path and client_key_path.

- Upload certificates to device (if offbox)

if not onbox:
    upload_certificates(dev, client_cert_path, client_key_path, log, targets_dict=targets_dict)
    if targets_dict["CA_server"]["CA_cert"]["fetch"] or targets_dict["CA_server"]["CA_cert"]["generate"]:
        upload_certificates(dev, ca_cert_path, ca_key_path, log, targets_dict=targets_dict)

- Uploads client certificate and key to the device using SCP.

- Uploads CA certificate and key only if it was fetched or generated automatically.

- Assumes manually uploaded CA certificates do not need re-uploading.

- Summary

- Check if certificates exist on the device.

- Check if the client certificate is about to expire.

- If certificates are missing or expiring:

  - Fetch or generate the CA certificate if needed.

  - Generate a new client certificate signed by the CA.

- Upload the client certificate and key to the device.

- Upload the CA certificate and key only if they were fetched/generated automatically.

This function essentially automates certificate lifecycle management: it detects missing/expiring certificates, renews them, and ensures the devices always have valid credentials.

## should_check_certs

This function determines whether it’s time to check certificates based on a schedule: every 5 days, and only during a certain time window.

- Function Signature

def should_check_certs():

Parameters: None Returns: True if certificates should be checked now, False otherwise.

- Get the current date and time

now = datetime.datetime.now()
current_time = now.strftime("%H:%M")

- now → the current date and time

- current_time → formatted as "HH:MM" (24-hour clock)

- Check if current time is in the allowed window

if "00:00" <= current_time <= "01:00":

- Only proceed if the current time is between midnight (00:00) and 1 AM (01:00).

- This ensures certificate checks happen during off-peak hours.

- Calculate the day of the month

days_passed = (now - now.replace(day=1)).days + 1

- Calculates the number of days since the start of the month.

- Adds 1 because replace(day=1) makes days_passed = 0 on the first day.

- Check if it’s a scheduled day

if days_passed % 5 == 1:
    return True

- Checks if the current day is “day 1 + multiples of 5” (1, 6, 11, 16, 21, 26, 31).

- Only these days are eligible for a certificate check.

- Return False otherwise

return False

- If the time is outside 00:00–01:00 or it’s not the scheduled day, the function returns False.

- Summary

- Get the current date and time.

- Only allow checks between midnight and 1 AM.

- Calculate how many days have passed since the start of the month.

- Only proceed if the current day is day 1, 6, 11, 16, 21, 26, or 31.

- Return True if both conditions are met, otherwise False.

Note: Note:

- This schedule ensures that certificate checks happen periodically every 5 days and during a safe time window to avoid interfering with production operations.

- The window is narrow (1 hour), so if this script is not running at that exact time, the check will be skipped.

## initialize_logging

This function sets up Python logging for your application, including custom log levels, output to console and files, and verbosity control.

- Function Signature

def initialize_logging(args):

Parameters:

- args → arguments object (e.g., from argparse)

  - args.verbose → controls logging verbosity

  - args.trace → if True, creates a detailed debug trace file

Returns:

- log → the initialized logging.Logger object

- Define a custom log level

LOG_NOTICE = 25
logging.addLevelName(LOG_NOTICE, "NOTICE")

- Adds a custom log level called NOTICE between WARNING (30) and INFO (20).

- Allows logging messages with medium importance.

- Add a notice method to Logger

def log_notice(self, message, *args, **kwargs):
    if self.isEnabledFor(LOG_NOTICE):
        self._log(LOG_NOTICE, message, args, **kwargs)
logging.Logger.notice = log_notice

- Adds a new Logger.notice() method to log at the NOTICE level.

- Define log levels list and capture warnings

LOG_LEVELS = [logging.ERROR, logging.WARNING, LOG_NOTICE, logging.INFO, logging.DEBUG]
logging.captureWarnings(True)

- List of possible log levels in increasing verbosity order.

- Converts Python warnings to logging messages automatically.

- Determine logging level based on verbosity

verbosity = min(args.verbose, len(LOG_LEVELS) - 1)
log_level = LOG_LEVELS[verbosity]

- Uses args.verbose to choose the logging level from LOG_LEVELS.

- Ensures it stays within valid range.

- Create logger and formatter

log = logging.getLogger()
formatter = logging.Formatter('%(asctime)s %(threadName)-10s %(name)s %(levelname)-8s %(message)s')
stderr = logging.StreamHandler()
stderr.setFormatter(formatter)
log.addHandler(stderr)

- Creates the root logger.

- Configures a formatter with timestamp, thread name, logger name, log level, and message.

- Adds a console handler (stderr) so logs appear on the terminal.

- Configure file logging

if args.trace:
    fh = logging.FileHandler('trace.log')
    fh.setLevel(logging.DEBUG)
    fh.setFormatter(formatter)
    log.setLevel(logging.DEBUG)
    log.addHandler(fh)
    stderr.setLevel(log_level)
elif onbox:
    fh = logging.handlers.RotatingFileHandler(LOG_FILENAME, maxBytes=10000000, backupCount=5)
    fh.setLevel(logging.INFO)
    fh.setFormatter(formatter)
    log.setLevel(logging.INFO)
    log.addHandler(fh)
    stderr.setLevel(log_level)
else:
    log.setLevel(log_level)

- Trace mode:

  - Writes all debug logs to trace.log.

  - Console shows logs according to verbosity.

- On-device (onbox) mode:

  - Writes logs to a rotating file (LOG_FILENAME).

  - Each file max 10 MB, keeps 5 backups.

- Off-device mode:

  - Logs only go to console, at the selected verbosity.

- Final info message

log.info('Logging modules initialized successfully')
print('Logging modules initialized successfully')

- Logs and prints a confirmation message to indicate logging is ready.

- Summary

- Adds a custom NOTICE log level.

- Captures Python warnings in the logger.

- Chooses log level based on verbosity argument.

- Sets up console logging and optionally file logging:

  - Trace file if args.trace

  - Rotating log file if onbox

- Prints/logs a message indicating logging is initialized.

- Returns the configured Logger object for use elsewhere.

Note: Notes / Suggestions

- Using RotatingFileHandler prevents logs from growing indefinitely.

- The custom NOTICE level is useful for mid-importance messages between INFO and WARNING.

- Could extend to per-module logging for finer control.

## get_previous_key_ids

This function reads previously stored key IDs from a JSON file. It is useful for tracking which cryptographic keys have already been used or generated.

- Function Signature

def get_previous_key_ids(log, name):

Parameters:

- log → logging object to write informational messages

- name → string used to format the JSON filename

Returns:

- A dictionary containing previous key IDs, or an empty dictionary {} if none exist.

- Try to read the JSON file

with open(KEYID_JSON_FILENAME.format(name), 'r') as openfile:
    return json.load(openfile)

- KEYID_JSON_FILENAME is a template path, e.g., '/var/home/admin/{}last_key.json'.

- format(name) replaces {} with the provided name.

- Reads the JSON file and returns its contents as a Python dictionary.

- Handle file not found

except FileNotFoundError:
    log.info(f"File to read previous keyId(s) not found")
    return {}

- If the JSON file doesn’t exist, likely because this is the first time the script runs:

  - Log an informational message.

  - Return an empty dictionary.

- Handle empty or invalid file

except ValueError:
    log.info(f"No previous keyId(s) found in file")
    return {}

- If the file exists but is empty or contains invalid JSON, json.load raises a ValueError.

- Logs a message and returns an empty dictionary.

- Summary

- Construct the JSON file path using name.

- Try to read the file and load previous key IDs into a dictionary.

- If the file doesn’t exist, log that it’s missing and return {}.

- If the file exists but is empty or invalid, log that no key IDs are found and return {}.

Note: Notes / Suggestions

- This function ensures the script doesn’t crash on first run.

- Returning {} is safe for subsequent operations that rely on previous keys.

- Could extend with a file lock if multiple processes might write to the same JSON file.

## save_key_ids

This function saves cryptographic key IDs to a JSON file for later retrieval. It complements the get_previous_key_ids function.

- Function Signature

def save_key_ids(key_dict, local_name):

Parameters:

- key_dict → dictionary containing key IDs to save

- local_name → string used to format the JSON filename

Returns: None (performs a file write)

- Construct the file path

KEYID_JSON_FILENAME.format(local_name)

- KEYID_JSON_FILENAME is a template path, e.g., '/var/home/admin/{}last_key.json'.

- Replaces {} with local_name to create a unique filename for each device or context.

- Write key IDs to the file

with open(KEYID_JSON_FILENAME.format(local_name), 'w+') as outfile:
    outfile.write(json.dumps(key_dict))

- Opens the file in write mode (w+), creating it if it doesn’t exist.

- Serializes the dictionary (key_dict) to JSON format using json.dumps.

- Writes the JSON data to the file.

- Overwrites any existing file content.

- Print confirmation

print(f'Saved the key IDs to the JSON file: {KEYID_JSON_FILENAME.format(local_name)}')

- Prints a message confirming that the key IDs were successfully saved.

- Useful for debugging or tracking the workflow.

- Summary

- Construct the JSON filename using local_name.

- Open the file (create it if it doesn’t exist).

- Convert the key dictionary into JSON and write it to the file.

- Print a confirmation message.

Note: Notes / Suggestions

- You could add logging instead of print for consistency with get_previous_key_ids.

- Consider atomic writes (writing to a temp file then renaming) to prevent data corruption if the script is interrupted.

- Using JSON ensures the file can easily be read back with get_previous_key_ids.

## fetch_kme_key

This function fetches cryptographic keys from a Key Management Entity (KME) using HTTPS requests with client and CA certificates for authentication.

- Function Signature

def fetch_kme_key(session, local_name, log, remote_mnmgt_add, kme_host, key_id, additional_slave_SAE_IDs=None):

Parameters:

- session → a requests.Session() object used for making HTTP requests

- local_name → the local device name (used to find its certificates)

- log → logging object

- remote_mnmgt_add → the master device or remote address for key retrieval

- kme_host → base URL of the KME server

- key_id → optional; if provided, fetch a specific key, otherwise fetch new keys

- additional_slave_SAE_IDs → optional list of additional devices for which to fetch keys

Returns:

- JSON object containing keys if the request succeeds, or None if it fails

- Determine certificate paths

if onbox:
    client_crt = CERTS_DIR + local_name + '.crt'
    client_key = CERTS_DIR + local_name + '.key'
    CLIENT_CERT = (client_crt, client_key)
    CA_CERT = CERTS_DIR + 'client-root-ca.crt'
else:
    client_crt = OFFBOX_CERTS_DIR + '/' + local_name + '.crt'
    client_key = OFFBOX_CERTS_DIR + '/' + local_name + '.key'
    CLIENT_CERT = (client_crt, client_key)
    CA_CERT = OFFBOX_CERTS_DIR + '/' + 'root.crt'

- Determines where to find client certificate, client key, and CA certificate depending on whether the script is running on-device (onbox) or off-device.

- These certificates are used for mutual TLS authentication.

- Make an HTTPS request to the KME

if key_id:
    response = session.get(f"{kme_host}/api/v1/keys/{remote_mnmgt_add}/dec_keys?key_ID={key_id}", verify=CA_CERT, cert=CLIENT_CERT, headers={"Content-Type": "application/json"})
else:
    response = session.get(f"{kme_host}/api/v1/keys/{remote_mnmgt_add}/enc_keys", verify=CA_CERT, cert=CLIENT_CERT, headers={"Content-Type": "application/json"})

- If key_id is provided → fetch a specific decryption key.

- If not → fetch new encryption keys from the KME.

- Uses CLIENT_CERT for client authentication and CA_CERT to verify the KME server.

- Sets Content-Type header to "application/json".

- Check the response

if response.status_code == 200:
    response_json = response.json()
    return response_json
else:
    print(f'Request failed with status code {response.status_code}')
    print(response.text)

- If the server responds with HTTP 200 OK → parse the JSON and return it.

- Otherwise → print the status code and response content for debugging.

- Handle exceptions

except requests.RequestException as e:
    log.error(f"KME request failed: {e.response.text}")
    print(f"KME request failed: {e.response.text}")
    return None

- If the HTTP request fails due to network issues, SSL errors, or server errors, log the error and return None.

- Summary

- Determine the client and CA certificate paths depending on whether the script is running on the device or off-device.

- Make a mutual TLS HTTPS request to the KME:

  - If key_id is given → fetch an existing key.

  - Otherwise → fetch new keys for the master device.

- Parse and return the JSON response if successful.

- Log and print errors if the request fails.

Note: Notes / Suggestions

- CLIENT_CERT and CA_CERT ensure mutual authentication between the client and the KME.

- The function currently prints a lot of debug info; consider using log.debug() instead of print for cleaner production logs.

- additional_slave_SAE_IDs is defined but not used; you could extend the request payload to include it if needed.

## check_and_apply_initial_config

This function checks whether the initial MACsec configuration is applied on a network device and applies it if it isn’t. It is intended to run only once on the first execution of the script.

- Function Signature

def check_and_apply_initial_config(dev, targets_dict, log):

Parameters:

- dev → a jnpr.junos.Device object representing the network device

- targets_dict → dictionary containing device details, interfaces, KME info, and CA server info

- log → logging object for informational and error messages

Returns: None (applies configuration directly to the device)

- Extract device information

device_name = dev.facts['hostname'].split('-re')[0]
device_ip = targets_dict[device_name]["ip"]
c_a = targets_dict["CA_server"]["c_a"]
interfaces = targets_dict[device_name]["interfaces"]
kme_name = targets_dict[device_name]["kme"]["kme_name"]
kme_ip = targets_dict[device_name]["kme"]["kme_ip"]
start_time = targets_dict["system"]["event_options"]["start_time"]

- Retrieves device name, IP, connectivity-association (CA) name, list of interfaces, KME host info, and event start time from targets_dict.

- Check if MACsec configuration already exists

config_check = dev.rpc.get_config(filter_xml=E.configuration(E.security(E.macsec(E('connectivity-association', E.name(c_a))))))
if config_check.find('.//name') is not None:
    log.info("Initial macsec configuration already applied on the device: {}.".format(device_name))
    return

- Uses Juniper RPC to check the device’s configuration.

- If the connectivity-association already exists, it logs a message and exits the function.

- Build the MACsec commands

initial_macsec_commands = [
    f"set security macsec connectivity-association {c_a} cipher-suite gcm-aes-xpn-256",
    f"set security macsec connectivity-association {c_a} security-mode static-cak",
    f"set security macsec connectivity-association {c_a} pre-shared-key ckn abcd1234abcd5678abcd1234abcd5678abcd1234abcd5678abcd1234abcd5678",
    f"set security macsec connectivity-association {c_a} pre-shared-key cak abcd1234abcd5678abcd1234abcd5678abcd1234abcd5678abcd1234abcd5678"
]

- Defines the basic MACsec CA configuration, including cipher, security mode, and pre-shared keys.

- Add interface-specific commands

for interface in interfaces:
    initial_macsec_commands.extend([
        f"set security macsec interfaces {interface} apply-macro qkd kme-ca false",
        f"set security macsec interfaces {interface} apply-macro qkd kme-host {kme_name}",
        f"set security macsec interfaces {interface} apply-macro qkd kme-port 443",
        f"set security macsec interfaces {interface} connectivity-association {c_a}",
        f"set security macsec interfaces {interface} apply-macro qkd kme-keyid-check true",
        f"set system static-host-mapping {device_name} inet {device_ip}"
    ])

- Configures each network interface to use MACsec, apply QKD macros, associate with KME, and map device IP.

- Add event options for on-device (onbox) mode

if onbox:
    initial_macsec_commands.extend([
        f"set event-options generate-event every10mins time-interval 600 start-time {start_time}",
        f"set event-options policy qkd events every10mins",
        f"set event-options policy qkd then event-script ETSIA_v3.1.0_Turkcell_Phase2_v1.py",
        f"set event-options event-script file ETSIA_v3.1.0_Turkcell_Phase2_v1.py python-script-user admin",
        f"set event-options traceoptions file script.log",
        f"set event-options traceoptions file size 10m",
    ])

- Sets up automated event generation every 10 minutes to run a QKD Python script.

- Configures logging and trace options for the event script.

- Apply the configuration

try:
    dev.timeout = 300
    with Config(dev) as cu:
        cu.lock()
        for command in initial_macsec_commands:
            cu.load(command, format='set')
        cu.commit()
        log.info("Initial macsec configuration applied successfully on the device: {}.".format(device_name))
except Exception as e:
    log.error(f'Initial macsec configuration commit failed: {e}')
finally:
    cu.unlock()
    time.sleep(60)

- Locks the device configuration.

- Loads all MACsec commands using set-style configuration.

- Commits the configuration.

- Unlocks the configuration and waits 60 seconds to allow changes to propagate.

- Logs success or error messages.

- Summary

- Extracts device details and KME info.

- Checks if MACsec configuration already exists; skips if present.

- Builds a list of set commands for MACsec and interface configuration.

- Adds event scheduling for running the QKD script if on the device.

- Locks the device configuration, loads all commands, commits, unlocks.

- Waits for 60 seconds to ensure configuration is applied.

- Logs success or failure.

Note: Notes / Suggestions

- Ensures the initial MACsec configuration is idempotent (applied only once).

- Uses QKD macros and static keys for secure communication.

- Event scripts automate recurring QKD key operations.

- Consider error handling for individual commands in large deployments.

## get_key_id_from_master

This function retrieves the Key ID file from the master device in a QKD (Quantum Key Distribution) setup. The Key ID file keeps track of which encryption keys have been generated or used by the master.

- Function Signature

def get_key_id_from_master(dev, log, targets_dict):

Parameters:

- dev → device object (Juniper Device) — not directly used here but passed for consistency

- log → logging object for informational messages (currently print is used)

- targets_dict → dictionary containing device information, roles, secrets, and paths

Returns:

- Path to the Key ID file on the local system, or None if it cannot be retrieved.

- Determine the master device

master_name = targets_dict["qkd_roles"]['master']
master_key_id_file = KEYID_JSON_FILENAME.format(master_name)

- Looks up the master device name from targets_dict.

- Constructs the Key ID filename for the master device using KEYID_JSON_FILENAME template.

- Handle off-device scenario (not onbox)

if not onbox:
    if os.path.exists(master_key_id_file):
        return master_key_id_file
    else:
        print('the master key id file does not exist')
        return None

- If running off-device:

  - Checks if the Key ID file already exists locally.

  - If yes → return its path.

  - If no → log a message and return None.

- Handle on-device scenario (onbox)

client = createSSHClient(master_name, username=targets_dict["secrets"]["username"], password=targets_dict["secrets"]["password"], port=22)
with SCPClient(client.get_transport()) as scp:
    scp.get(remote_path=master_key_id_file, local_path=master_key_id_file, preserve_times=True)

- Connects to the master device over SSH using createSSHClient.

- Uses SCPClient to copy the master Key ID file from the master device to the local system.

- preserve_times=True keeps the original file timestamps.

- Error handling

except SCPException as e:
    print(f'SCP get exception error: {e}')
    return None

- If the SCP transfer fails (network issues, missing file, permission errors), logs the exception and returns None.

- Return local path

return master_key_id_file

- Returns the path to the retrieved Key ID file on the local system.

- This file can then be read to get the master’s current key IDs.

- Summary

- Determine the master device and its Key ID filename.

- If running off-device (not onbox):

  - Check if the file exists locally → return path or None.

- If running on-device (onbox):

  - Connect to the master over SSH.

  - Copy the Key ID file via SCP.

  - Return the local path.

- Logs any errors encountered during SCP transfer.

Note: Notes / Suggestions

- Currently uses print() for messages; consider using log.info() or log.error() for consistency.

- Could add retries or timeout handling for SCP in case of transient network issues.

- Ensures that all devices in the QKD network can access the master’s Key ID to stay synchronized.

## process

The process function is the core logic for applying MACSEC keys to a Junos device. It handles fetching keys from the KME (Key Management Entity), updating the device configuration, and storing the key IDs for future runs.

- Function Signature

def process(dev, targets_dict, log):

Parameters:

- dev: Juniper device object from jnpr.junos.Device

- targets_dict: Dictionary containing all device info, roles, credentials, interfaces, and KME info

- log: Logger object for logging events

- Get MACSEC configuration

conf_filter = E.configuration(E.security(E.macsec()))
config_xml = dev.rpc.get_config(filter_xml=conf_filter)
macsec_xml = config_xml.find('security/macsec')

- Fetches the current MACSEC configuration from the device using Junos XML RPC.

- Prepares the configuration object for further modifications.

- Prepare key fetching session

session = requests.Session()
new_key_dict = {}
qkd_macsec_xml = E.macsec()
commit = False

- Creates a requests.Session for communicating with the KME API.

- new_key_dict will store the Key IDs fetched for this device.

- qkd_macsec_xml is a placeholder for new MACSEC configuration.

- commit flag indicates whether to apply changes.

- Determine CA name and local hostname

ca_name = macsec_xml.findtext('connectivity-association/name')
local_name = dev.facts['hostname'].split('-re')[0]
kme_host = targets_dict[local_name]["kme"]["kme_name"]

- Extracts the connectivity association (CA) name from the MACSEC config.

- Determines the local hostname (removes -re suffix).

- Finds the KME host associated with this device.

- Fetch Key from KME

- Master Device Logic:

if targets_dict["qkd_roles"]['master'] == local_name:
    remote_mnmgt_add = targets_dict["qkd_roles"]['slave']
    r = fetch_kme_key(session, local_name, log, remote_mnmgt_add, kme_host, key_id=None)
    key = r['keys'][0]
    new_key_dict[local_name] = key['key_ID'].strip()

- If this device is the Master, fetch a new encryption key from the KME for the Slave device.

- Updates new_key_dict with the received Key ID.

- Slave Device Logic:

else:
    remote_mnmgt_add = targets_dict["qkd_roles"]['master']
    # Retry loop for master key
    master_key_id_file = get_key_id_from_master(dev, log, targets_dict)
    master_key_dict = get_previous_key_ids(log, remote_mnmgt_add)
    new_key_dict[local_name] = master_key_dict[remote_mnmgt_add]

- If the device is a Slave, fetch the Key ID from the Master device.

- Retries every 5 seconds until a new Key ID is available.

- Updates new_key_dict with the Master’s Key ID.

- Update MACSEC CAK and CKM

qkd_ca_xml.find('pre-shared-key/ckn').text = CKN_PREFIX + uuid.UUID(key['key_ID']).hex
qkd_ca_xml.find('pre-shared-key/cak').text = str(base64.b64decode(key['key']).hex())[:64]
qkd_macsec_xml.append(qkd_ca_xml)
commit = True

- Updates the connectivity association key name (CKN) and connectivity association key (CAK) in the configuration XML.

- Appends the new MACSEC config to qkd_macsec_xml.

- Sets commit = True so it will be applied.

- Root Authentication

root_auth_xml = E.system(
    E("root-authentication",
        E("encrypted-password", targets_dict[local_name]["root_enc_pass"])
    )
)

- Prepares root authentication XML (encrypted password) to be included in the configuration if needed.

- Commit Configuration

qkd_config_xml = E.configuration(E.security(qkd_macsec_xml))
with Config(dev) as cu:
    cu.lock()
    cu.load(qkd_config_xml, format='xml', merge=True)
    cu.commit()
    cu.unlock()

- Converts XML to a Junos configuration object.

- Locks the configuration database.

- Loads the new configuration and merges it with existing settings.

- Commits the configuration changes to the device.

- Unlocks the configuration database.

- Logs success or failure of the commit.

- Save Key IDs

save_key_ids(new_key_dict, local_name)

- Saves the updated Key IDs to a local JSON file for future reference.

- Optional Verification

mka_session_info_xml = dev.rpc.get_mka_session_information({'format':'text'}, summary=True)

- Fetches MKA (MACsec Key Agreement) session information for debugging or verification.

- Summary

- Fetch current MACSEC configuration from the device.

- Determine if the device is Master or Slave in the QKD network.

- Master: request a new key from the KME for Slave.

- Slave: fetch the key ID from the Master device. Retry until a new key is available.

- Update MACSEC keys (CKN and CAK) in the device configuration.

- Commit the configuration changes to the device.

- Save the new key IDs locally for future runs.

- Optionally, fetch MACSEC session information for verification.

Note: Notes / Suggestions

- Currently prints a lot of debug information — you could replace most print statements with log.debug for cleaner logs.

- The retry logic for Slave devices ensures synchronization between Master and Slave, but it might hang if the Master never updates the key file.

- commit = True is controlled dynamically based on whether new keys are available.

## get_args

The get_args() function is responsible for defining and parsing command-line arguments for the script. It uses Python’s argparse module.

- Function Signature

def get_args():

- No input parameters.

- Returns a Namespace object containing the parsed arguments.

- Create Argument Parser

parser = argparse.ArgumentParser()

- Creates an ArgumentParser object to handle command-line arguments.

- This object will automatically generate help messages.

- Define Arguments

- Threads (-t / --threads)**

parser.add_argument("-t", "--threads", type=int, help="Number of threads to use")

- Optional argument to specify how many threads the script should run.

- Example usage:

python script.py -t 5

- Verbose (-v / --verbose)**

parser.add_argument('-v','--verbose', default=0, action='count', help="increase verbosity level")

- Optional flag to increase the verbosity of logs.

- Can be specified multiple times (-vv for more verbose).

- Default is 0 (no extra verbosity).

Example:

python script.py -vv

- action='count' automatically counts how many times -v is used.

- Trace (-tr / --trace)**

parser.add_argument('-tr','--trace', action='store_true', help="dump debug level logs to trace.log file")

- Optional flag to enable trace logging.

- Stores a boolean True if present on the command line, otherwise False.

Example:

python script.py --trace

- Parse Arguments

return parser.parse_args()

- Reads the command-line arguments provided when the script is run.

- Returns them as a Namespace object, which can be accessed like:

args = get_args()
print(args.threads)  # Number of threads
print(args.verbose)  # Verbosity level
print(args.trace)    # True/False

- Summary

- Prepares the script to accept command-line options.

- Supports threads, verbosity, and trace logging flags.

- Returns a convenient object (args) containing all user-specified options.

## main

The main() function orchestrates the full workflow for configuring devices with QKD MACSEC and fetching keys from KME.

- Parse Command-Line Arguments

args = get_args()

- Reads arguments like:

  - Number of threads to use (--threads)

  - Verbosity level (-v)

  - Trace mode (--trace)

- Initialize Logging

log = initialize_logging(args)

- Sets up logging based on verbosity or trace mode.

- Logs to console, file, or rotating log file depending on environment (onbox or offbox).

- Define Target Devices and System Info

targets_dict = {...}

- Contains:

  - System-wide settings (e.g., max threads, start time for events)

  - Secrets (SSH username/password)

  - CA server info (certificate fetch/generate options, paths)

  - QKD roles (master, slave, additional devices)

  - Device-specific data (IP, root password, interfaces, KME details)

Essentially, this is your entire network and KME configuration in a single dictionary.

- Offbox vs Onbox Workflow

if not onbox:
    # Offbox processing
elif onbox:
    # Onbox processing

- Offbox: Script runs from a separate host (not on the devices themselves).

  - It calculates threads and distributes devices across them.

  - Launches req_thread for each set of devices.

  - Waits for all threads to finish (t.join()).

- Onbox: Script runs directly on a device.

  - Connects to the device using Device().

  - Runs the configuration steps synchronously:

    - check_and_apply_initial_config() — applies initial MACSEC configuration if not already present.

    - process() — fetches keys from KME and updates MACSEC configuration.

  - Uses profiling (prof.start / prof.stop) to measure execution time for each step.

- Threading Logic (Offbox)

- Creates a list of devices to process in threads:

dlist = [master, slave, additional slaves]

- Splits the list according to the number of threads.

- Calls req_thread() for each batch, which is decorated with @background to run in a separate thread.

- Waits for all threads to finish before continuing.

- Onbox Logic

- Runs everything sequentially on the device.

- Uses profiling to measure time taken by:

  - Certificate renewal (if needed)

  - Initial MACSEC configuration

  - Fetching keys and updating MACSEC configuration

- Error Handling

- Logs and prints exceptions if SSH or device operations fail.

- Summary

- Purpose: Automate QKD MACSEC configuration on multiple devices.

- Supports two modes: Onbox (direct device) or Offbox (external host with threading).

- Workflow:

  - Parse CLI args

  - Initialize logging

  - Load device and KME configuration

  - Apply initial MACSEC config

  - Fetch keys from KME

  - Update device configuration

  - Save new Key IDs for tracking

Below is the “How to Run” section. It includes clear steps, examples for both onbox and offbox, and environment expectations.

# How to Run the Script

This section explains how to run the QKD–MACsec automation script in both offbox (management host) and onbox (directly on a router) modes, with real and practical examples.

## Offbox Mode

- Recommended for managing many devices

You need a Linux/BSD/Unix host with:

- Python 3.7+

- Juniper PyEZ (junos-eznc)

- paramiko, scp, pyOpenSSL

- SSH reachability to each device

- The targets.yaml or targets.json configuration file

## Onbox Mode

- Script runs directly on JunOS

You need:

- JunOS with event-options and Python enabled

- Script copied to /var/db/scripts/event/

- Proper permissions for python-script-user

## Script Usage

The script supports the following command-line options:

| Option | Description |
| --- | --- |
| -t, --threads | Number of parallel threads (offbox only) |
| -v | Increase verbosity (can be used multiple times) |
| -tr, --trace | Log debug-level output to trace.log |

## Running in Offbox Mode

### Basic Example

Run the script with default settings:

python3 qkd_macsec.py

If no thread count is provided, the script uses 1 thread.

### Using Multiple Threads

To run on 10 devices in parallel:

python3 qkd_macsec.py --threads 10

This will:

- Spawn 10 worker threads

- Each thread executes req_thread()

- Each device goes through the full workflow:

  - Initial MACsec config (if not applied)

  - Certificate validation/renewal

  - Key ID retrieval (master → slave)

  - KME key fetch

  - MACsec key/CA update

  - Key ID persistence

### Verbose Output

More -v flags increase log detail.

python3 qkd_macsec.py -vv

- -v: INFO

- -vv: DEBUG

- -vvv: VERY verbose

### Write Debug Information to trace.log

python3 qkd_macsec.py --trace

This enables debug-level trace logging in:

./trace.log

### Full Example with All Options

python3 qkd_macsec.py --threads 5 -vv --trace

Meaning:

- Use 5 threads

- Verbose debug messages to console

- Full debug tracing logged to trace.log

## Running in Onbox Mode

- Through JunOS Event Script

The script can also run as an event-triggered script using event-options.

### Copy the Script to the Device

Place the file in:

/var/db/scripts/event/qkd_macsec.py

Make sure permissions allow execution:

chmod 755 /var/db/scripts/event/qkd_macsec.py

### Configure Event-Options

- Example

Example JunOS config:

set event-options generate-event every10mins time-interval 600 start-time 00:00:00
set event-options policy qkd events every10mins
set event-options policy qkd then event-script qkd_macsec.py
set event-options event-script file qkd_macsec.py python-script-user admin
set event-options traceoptions file script.log
set event-options traceoptions file size 10m

This will:

- Trigger the script every 10 minutes

- Run it as user admin

- Log to /var/log/script.log

### Manual Onbox Execution

You can manually run the script on the router:

python3 /var/db/scripts/event/qkd_macsec.py

This follows the same device workflow:

- Certificate validation/renewal

- Key ID fetch (from master or local file)

- KME communication

- MACsec updates

## File Outputs

During execution, the script generates:

### Key ID file

<hostname>_keyID.json

### Certificates

ca_cert.pem
ca_key.pem
client_cert.pem
client_key.pem

### Logs

- script.log (onbox trace)

- trace.log (offbox debug trace if enabled)

- Console output based on -v flags

## Typical Workflow Example

### Scenario

You have 3 devices:

- PE1 (master)

- PE2 (slave)

- PE3 (slave)

And you want to configure all of them at once with QKD–MACsec.

### Run

python3 qkd_macsec.py --threads 3 -vv --trace

- The script will:

- Connect to PE1, PE2, PE3

- Apply initial MACsec config (if needed)

- Validate certificates; renew if needed

- Fetch CA and client certificates

- PE1 fetches key from KME → saves key ID

- PE2/PE3 retrieve master key ID

- Each device fetches its KME key

- Final MACsec config is applied

- All key IDs saved for next iteration

### Confirming Device Configuration

On a JunOS device, check MACsec settings:

show security macsec
show security certificates local
show log script.log

Check event triggers:

show event-options policy
show event-options event-script

# Appendix

- A complete targets.yaml example**

- A full Troubleshooting section**

## targets.yaml

- Example Configuration File

This file defines all devices, roles (master/slave), KME endpoints, credentials, CA behavior, and system parameters.

system:
  maxthreads: 4
  event_options:
    start_time: "2025-03-23.13:00:00"

secrets:
  username: "admin"
  password: "admin123!"

CA_server:
  CA_cert:
    fetch: false
    generate: false
  c_a: "CA_basic"
  ca_server_ip: "192.168.10.10"
  ca_path: "/opt/ca/"
  ca_cert_name: "root_ca_cert.pem"
  ca_key_name: "root_ca_key.pem"
  ca_user: "caadmin"
  ca_pass: "capassword12!"

qkd_roles:
  master: "acx-1"
  slave: "acx-2"
  additional_slave_SAE_IDs: ["acx-3", "acx-4"]

acx-1:
  root_enc_pass: "$6$3eHulK1c$Yq.kaamV8hcuviwRebQI4gUMRSOGVIiBN8o/QTw7sfZ4GCfExd3TjyuUrsyrgfoBW3xNQVT5/gtGg6.S09okg0"
  ip: "9.173.8.201"
  interfaces:
    - "et-0/0/20:2"
  kme:
    kme_name: "https://idq-1"
    kme_ip: "9.173.9.102"

acx-2:
  root_enc_pass: "$6$bOeXyUQ7$oefu0aDycBhyLGDE.TCExBrdVkYOhg2IOesMVwRQvid9iDpMzwm5yZPvYhKlBu3sZ0YbHBAH0ro5SQWTnscWf."
  ip: "9.173.8.202"
  interfaces:
    - "et-0/0/20:2"
  kme:
    kme_name: "https://idq-2"
    kme_ip: "9.173.9.103"

acx-3:
  root_enc_pass: "$6$someotherpass$morehashdata"
  ip: "9.173.8.203"
  interfaces:
    - "et-0/0/20:2"
  kme:
    kme_name: "https://idq-3"
    kme_ip: "9.173.9.104"

acx-4:
  root_enc_pass: "$6$randompass$hashxxxyyy"
  ip: "9.173.8.204"
  interfaces:
    - "et-0/0/20:2"
  kme:
    kme_name: "https://idq-4"
    kme_ip: "9.173.9.105"

# Troubleshooting Guide

This section helps diagnose common issues when running the QKD–MACsec automation script.

## Script Fails to Connect to Device and SSH Errors

- Symptoms

- TimeoutError

- Authentication failed

- SSHException: Error reading SSH protocol banner

- Causes

- Wrong username/password

- Device not reachable

- SSH disabled or filtered by firewall

- Fix

ssh admin@<device-ip>

If SSH fails manually → fix network or credentials.

Ensure device allows Python script execution:

set system scripts language python3

## KeyID JSON file not found

- Message

File to read previous keyId(s) not found

This is not an error during the first run.

- Fix

None required — the script will create <device>_keyID.json automatically.

## MACsec Not Coming Up After Script Execution

- Checks

- Verify MACsec status:

show security macsec

- Verify key exchange status:

show security macsec connectivity-association

- Check KME communication:

show log messages | match kme

- Common Causes

- Key mismatch between master and slaves

- Wrong interface in targets.yaml

- KME IP unreachable

- CA certificate mismatch

## Certificate Creation or Upload Fails

- Symptoms

- Missing certificate fields

- “invalid certificate format”

- Upload fails on device

- Fix Checklist

- Ensure OpenSSL is installed (offbox)

- Ensure CA paths are correct

- Ensure CA and client keys are readable by script

- Check certificate validity:

openssl x509 -in client_cert.pem -text

## SCPException When Fetching Master KeyID File

- Causes

- Wrong master hostname

- Wrong file path

- SCP blocked

- Fix

Check remote file existence:

file list /var/home/admin/acx-1_keyID.json

Verify scp manually:

scp admin@acx-1:/var/home/admin/acx-1_keyID.json .

## Offbox mode: threads Start but Nothing Happens

- Causes

- targets.yaml has no devices

- Threads created but req_thread() exits early due to local error

- Logging verbosity too low to see details

- Fix

Run with high verbosity:

python3 qkd_macsec.py --threads 5 -vvv

Check trace.log if enabled.

## Onbox Mode: Script Not Triggering

- Causes

- Event-options misconfigured

- Script not in correct directory

- Wrong owner/permissions

- Fix Steps

- Verify script placement:

/var/db/scripts/event/<your-script>.py

- Verify JunOS event options:

show event-options policy
show event-options event-script

- Check logs:

show log script.log

## KME Not Returning Key

- Symptoms

- “key_id not found”

- “failed to fetch key”

- “400/403 from KME”

- Fix

- Verify KME is reachable:

curl https://<kme-ip>/status

- Validate TLS certificates

- Ensure device hostname matches KME policy

## Device Fails at check_and_apply_initial_config

- Symptoms

- Interface not found

- MACsec configuration rejected

- Fix

Verify interfaces in targets.yaml match device:

show interfaces terse | match et-0/0/20

## Script Crashes with Python Exceptions

- Common Causes

- Missing dependencies

- Invalid path

- Typo in hostname

- Corrupted JSON keyID file

- Fix

Delete corrupted keyID file:

rm <device>_keyID.json

Re-run script.

# Python Profiler Class

This document describes a Python 3 profiler script that allows you to measure and log the execution time of tasks (or code sections) in your Python code, in milliseconds. It’s designed to help you see how long different parts of a script take to run, including counts, cumulative time, min/max, and averages. The profiler includes a context manager interface for clean and hierarchical task timing.

## Overview

The profiler allows you to:

- Measure execution time of named tasks (allows timing named tasks with start/stop calls).

- Collects stats: track count, cumulative time, average, minimum, and maximum durations.

- Writes a detailed profiling log to a Log file, in a human-readable format.

- Profile and support nested tasks (Outer → Inner → Loop) with automatic hierarchical timing.

- Useful for benchmarking parts of a Python program without external libraries.

The script is fully compatible with Python 3 and can be used as a module in other scripts.

## Features

- Task timing using start(task) and stop(task) methods.

- Verbose logging of task start/stop events.

- Automatic statistics: cumulative time, min/max time, average, and iteration counts.

- Annotate the profiling log with custom text.

- Context manager interface for clean nested profiling:

with profiler.task("MyTask"):
    # code to profile

- Clean, modern Python 3 implementation using f-strings and with open(...).

- Output summary sorted by cumulative execution time.

## Installation

Simply copy the profiler.py file into your project. No external dependencies are required. The script uses only Python standard library modules (time, datetime, contextlib).

## Usage

### Basic start/stop profiling

from profiler import Profile
import time

profiler = Profile(file_path="profile.log", verbose=True)

profiler.start("TaskA")
time.sleep(1)  # Simulate work
profiler.stop("TaskA")

profiler.close()

### Recommended context manager usage

from profiler import Profile
import time

profiler = Profile(file_path="profile.log", verbose=True)

with profiler.task("Outer"):
    time.sleep(1)  # Some code to profile

profiler.close()

- Using the context manager is cleaner and automatically handles start() and stop().

- Supports nested tasks without manually tracking start/stop.

### Nested loops example

This example demonstrates nested profiling for loops, similar to the original test case:

from profiler import Profile
import time

profiler = Profile(file_path="prof.test", verbose=True)
profiler.annotate("Unit test of profiler class with context manager")

with profiler.task("Outer"):
    for i in range(10):
        with profiler.task("Inner"):
            for j in range(5):
                with profiler.task("Loop"):
                    print("Processing inner loop work...")
                    time.sleep(0.1)  # Simulate work

profiler.close()

- Tasks "Outer", "Inner", and "Loop" are tracked separately.

- Automatically calculates count, cumulative, min/max, and average times for each task.

## Profiler Output

A typical profiling log looks like this:

Profiler started: 2025-11-27 12:00:00
Unit test of profiler class with context manager
Starting task "Outer"
Starting task "Inner"
Starting task "Loop"
Finished task "Loop" iter 1 elapsed 100ms (0:00:00.100000)
...

=== Summary ===
Task: Loop | Count: 50 | Cumulative: 5000ms (0:00:05) | Avg: 100.00ms | Min: 100ms | Max: 100ms
Task: Inner | Count: 10 | Cumulative: 5000ms (0:00:05) | Avg: 500.00ms | Min: 500ms | Max: 500ms
Task: Outer | Count: 1 | Cumulative: 5000ms (0:00:05) | Avg: 5000.00ms | Min: 5000ms | Max: 5000ms

GLOBAL execution elapsed: 6000ms (0:00:06)
##################################################

- Shows task counts, cumulative execution time, average, min/max, and global execution time.

- Nested tasks are automatically tracked.

## Advanced Notes

- Enable/Disable profiling: Pass enabled=False when creating the profiler to temporarily disable profiling.

- Verbose mode: Set verbose=True to log start/stop events to the profiling file.

- Custom annotations: Use profiler.annotate("My note") to add custom messages in the log.

- File management: The profiler appends to the specified log file; previous logs are preserved unless you manually delete the file.

# Appendix

## Glossary of MACsec Concepts

- AN - Association Number A two-bit value concatenated with a Secure Channel Identifier to identify a Secure Association. Used to easily identify if a Secure Association Key (SAK) has rolled over.

- CA - Connectivity Association A long-term relationship between endpoints that share common MACsec parameters, such as a key. On a router-to-router link, a CA would contain two unidirectional secure channels.

- CAK - Connectivity Association Key A long-term key either configured statically or assigned dynamically via RADIUS.

- CKN - Connectivity Association Key Name A hexadecimal Key Name value that identifies a CAK. It can be a short (but an even number of characters) set of values shared by participating endpoints when agreeing on which CAK to use.

- ICV - Integrity Check Value A field similar to the CRC, which protects the destination and source MAC addresses of the Ethernet packet and everything in between. The Integrity Check Value (ICV) validates that the MACsec packet is correct in both encrypted and unencrypted MACsec sessions.

- MKA - MACsec Key Agreement A protocol used between MACsec peers to synchronize keys. MKA securely encrypts and distributes the SAK between peers, handles the rollover of keys, and includes a built-in polling interval to allow for key server redundancy. MKA uses the EAP over LAN EtherType of 0x888e.

- SA - Secure Association A secure channel has a succession of secure associations (SAs), each with its own Secure Association Number (AN). A Secure Association Key (SAK) is unique for each SA and is derived from the long-term CAK. The Junos OS rolls over to the next SA either during hitless rollovers and keychain usage or when packet sequence numbers would roll over.

- SAK - Secure Association Key A key derived from the Connectivity Association Key (CAK) and relevant only for the duration of the Secure Association between two peers. The SAK is securely distributed between MACsec peers.

- SC - Secure Channel A unidirectional channel from one host to one or more hosts on the same broadcast domain. Typically, on point-to-point links, an SC has one recipient and includes a succession of secure associations (SAs).

- SCI - Secure Channel Identifier A concatenation of the Secure Channel sender’s MAC address and a port ID. The SCI remains constant through secure association rollovers within a secure channel.

- SecTAG - MAC Security Tag The MACsec header, starting with the MACsec Ethertype of 0x88e5, follows the standard destination and source MAC fields on any Ethernet packet.

- XPN - Packet Number and eXtended Packet Number A sequence number increasing by one for each packet sent within an SA. The initial MACsec specification used a 32-bit identifier, necessitating frequent SA renegotiation on high-speed links. eXtended Packet Numbering (XPN) increases the packet sequence number space to 64-bits.

| Term | Meaning |
| --- | --- |
| AN – Association Number | A two-bit value concatenated with the SCI to identify a Secure Association. Allows peers to detect SAK rollover. |
| CA – Connectivity Association | A long-term relationship between endpoints that share MACsec parameters. Contains two unidirectional secure channels. |
| CAK – Connectivity Association Key | A long-term key, either static or distributed by RADIUS. |
| CKN – Connectivity Association Key Name | A hexadecimal identifier for a CAK. Must be an even number of characters. |
| ICV – Integrity Check Value | A CRC-like field that protects the Ethernet header and payload. Ensures authenticity and integrity of MACsec traffic. |
| MKA – MACsec Key Agreement | Protocol that synchronizes peers, exchanges SAKs securely, handles SAK rollover, and supports redundancy. Uses EtherType 0x888e. |
| SA – Secure Association | A single instantiation of secure communication inside an SC. Each SA uses a unique SAK derived from the CAK. |
| SAK – Secure Association Key | A short-term key derived from the CAK. Used only within one Secure Association. Securely distributed via MKA. |
| SC – Secure Channel | A unidirectional channel composed of one or more SAs. On P2P links, each SC typically has one receiver. |
| SCI – Secure Channel Identifier | Concatenation of the sender’s MAC address and a port ID. Remains constant across SAK rollovers. |
| SecTAG – MAC Security Tag | The MACsec header beginning at EtherType 0x88e5, immediately after the source MAC field. |
| XPN – Extended Packet Number | A 64-bit packet counter replacing the original 32-bit PN to prevent frequent SA renegotiations. |

## Imported Packages and Modules Synthesis

### requests

- Purpose: A popular HTTP library in Python used to send HTTP requests (GET, POST, etc.) to web servers and handle the responses.

- Common Usage: Making REST API calls, downloading content from the web, and interacting with web services.

### jnpr.junos (Device, Config)

- Purpose: Part of the PyEZ library provided by Juniper Networks for automating and managing Junos devices (e.g., routers, switches).

- Common Usage:

  - Device: Used to establish a connection to a Junos device.

  - Config: Used to manage the configuration on a Junos device (loading, committing changes).

### paramiko (SSHException)

- Purpose: A Python library used to handle SSH connections and execute commands on remote machines securely.

- Common Usage: Establishing SSH connections, executing remote commands, handling SSH-related exceptions.

### scp (SCPClient, SCPException)

- Purpose: A module that works with paramiko to copy files to and from a remote machine using the SCP protocol.

- Common Usage: Securely transferring files over SSH between a local and a remote machine.

### lxml (etree, E)

- Purpose: A library for processing XML and HTML documents.

- Common Usage:

  - etree: Used to parse and interact with XML data structures.

  - E: A convenient way to build XML documents using a more Pythonic syntax.

### argparse

- Purpose: A module for parsing command-line arguments passed to a Python script.

- Common Usage: Define and handle command-line arguments for scripts, making them more flexible and user-friendly.

### threading (Thread, Lock)

- Purpose: The threading module provides tools for working with threads in Python, allowing for concurrent execution.

- Common Usage:

  - Thread: Create and manage threads.

  - Lock: Synchronize threads to prevent race conditions.

### logging (handlers)

- Purpose: A standard module for logging events, errors, and debug information in Python applications.

- Common Usage:

  - Setting up logging configurations.

  - handlers: Manage log file rotation, logging to different destinations, etc.

### json

- Purpose: A module for working with JSON data, a common data interchange format.

- Common Usage: Serializing Python objects to JSON format and deserializing JSON strings back into Python objects.

### copy

- Purpose: Provides functions to create shallow or deep copies of Python objects.

- Common Usage: Duplicating objects when you need to modify them without altering the original.

### base64

- Purpose: A module for encoding and decoding data in Base64, a binary-to-text encoding scheme.

- Common Usage: Encoding binary data for transmission over media that are designed to handle textual data.

### uuid

- Purpose: A module for generating universally unique identifiers (UUIDs).

- Common Usage: Creating unique identifiers for objects, such as in distributed systems.

### re

- Purpose: A module for working with regular expressions in Python.

- Common Usage: Searching, matching, and manipulating strings based on patterns.

### subprocess

- Purpose: A module for spawning new processes, connecting to their input/output/error pipes, and obtaining their return codes.

- Common Usage: Running shell commands from within a Python script.

### os

- Purpose: A module providing a way of interacting with the operating system, including file and directory management.

- Common Usage: Working with the file system, environment variables, and executing system commands.

### OpenSSL (crypto)

- Purpose: A wrapper around a subset of the OpenSSL library, which provides tools for cryptography, including managing certificates.

- Common Usage: Generating and managing SSL/TLS certificates, encryption/decryption.

### datetime

- Purpose: A module for working with dates and times in Python.

- Common Usage: Creating, manipulating, and formatting date/time objects.

### schedule

- Purpose: A lightweight job scheduling library for Python, which allows running tasks at specific intervals.

- Common Usage: Scheduling and managing periodic tasks in a Python application.

### Profile

- Purpose: A module for profiling and analyzing the performance of Python code.

- Common Usage: Measuring and optimizing the execution time of Python functions and scripts.

### time

- Purpose: A module providing various time-related functions.

- Common Usage: Pausing execution, getting the current time, and measuring intervals.

## Sample logs

### Offbox parallel execution

- (4 devices under test: mx304-11, mx304-12, mx10008-23 and mx10008-24)

administrator@jump:~$ python3 ETSIA_v3.0.py 
onbox False
Logging modules initialized successfully
Offbox approach taken
start check_and_apply_initial_config
start check_and_apply_initial_config
start check_and_apply_initial_config
start check_and_apply_initial_config
stop check_and_apply_initial_config
start process
stop check_and_apply_initial_config
start process
local_name: mx304-11
mx304-11 is Master
local_name: mx304-12
mx304-12 is Slave
local_name: mx304-12 last_key_dict: {'mx304-12': 'c107281c-9d04-4900-b731-896ab8f3a6c2'}
local_name: mx304-12 master_key_dict: {'mx304-11': 'c107281c-9d04-4900-b731-896ab8f3a6c2'}
local_name: mx304-12 Same KeyId: c107281c-9d04-4900-b731-896ab8f3a6c2, Retrying 1
stop check_and_apply_initial_config
start process
stop check_and_apply_initial_config
start process
local_name: mx10008-24
mx10008-24 is Slave
local_name: mx10008-24 last_key_dict: {'mx10008-24': 'c107281c-9d04-4900-b731-896ab8f3a6c2'}
local_name: mx10008-24 master_key_dict: {'mx304-11': 'c107281c-9d04-4900-b731-896ab8f3a6c2'}
local_name: mx10008-24 Same KeyId: c107281c-9d04-4900-b731-896ab8f3a6c2, Retrying 1
local_name: mx10008-23
mx10008-23 is Slave
local_name: mx10008-23 last_key_dict: {'mx10008-23': 'c107281c-9d04-4900-b731-896ab8f3a6c2'}
local_name: mx10008-23 master_key_dict: {'mx304-11': 'c107281c-9d04-4900-b731-896ab8f3a6c2'}
local_name: mx10008-23 Same KeyId: c107281c-9d04-4900-b731-896ab8f3a6c2, Retrying 1
local_name: mx304-11: {'keys': [{'key_ID': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19', 'key': 'nK45tzFFjDA42/1Ko5B6Hz2wi35gr72VsiHyVbMD9X9qxRbHOKe8D4NpHUy1nX96JeVF4xOp0qOiDLH9ldIb4Q=='}]}
KME: [GET] Get Keys API: https://kme-5.acct-1286.etsi-qkd-api.qukaydee.com/api/v1/keys/mx304-12/enc_keys
KME: [GET] Get Keys API: for mx304-12 {'keys': [{'key_ID': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19', 'key': 'nK45tzFFjDA42/1Ko5B6Hz2wi35gr72VsiHyVbMD9X9qxRbHOKe8D4NpHUy1nX96JeVF4xOp0qOiDLH9ldIb4Q=='}]}
Received KeyId:d9b6fa63-dd88-47d2-8afe-94fcecf40d19
ckn abcd1234abcd5678abcd1234abcd5678d9b6fa63dd8847d28afe94fcecf40d19
cak 9cae39b731458c3038dbfd4aa3907a1f3db08b7e60afbd95b221f255b303f57f
local_name: mx304-11 commit True
Configuration <configuration>
  <security>
    <macsec>
      <connectivity-association>
        <name>CA_basic</name>
        <cipher-suite>gcm-aes-xpn-256</cipher-suite>
        <security-mode>static-cak</security-mode>
        <pre-shared-key>
          <ckn>abcd1234abcd5678abcd1234abcd5678d9b6fa63dd8847d28afe94fcecf40d19</ckn>
          <cak>9cae39b731458c3038dbfd4aa3907a1f3db08b7e60afbd95b221f255b303f57f</cak>
        </pre-shared-key>
      </connectivity-association>
    </macsec>
  </security>
</configuration>

========================== script run SUCCESS =============================
{'mx304-11': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19'}
Saved the key IDs to the JSON file: /home/administrator/mx304-11_last_key_test.json
local_name: mx304-12 last_key_dict: {'mx304-12': 'c107281c-9d04-4900-b731-896ab8f3a6c2'}
local_name: mx304-12 master_key_dict: {'mx304-11': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19'}
local_name: mx304-12 Previous keyID: c107281c-9d04-4900-b731-896ab8f3a6c2
local_name: mx10008-24 last_key_dict: {'mx10008-24': 'c107281c-9d04-4900-b731-896ab8f3a6c2'}
local_name: mx10008-24 master_key_dict: {'mx304-11': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19'}
local_name: mx10008-24 Previous keyID: c107281c-9d04-4900-b731-896ab8f3a6c2
local_name: mx10008-23 last_key_dict: {'mx10008-23': 'c107281c-9d04-4900-b731-896ab8f3a6c2'}
local_name: mx10008-23 master_key_dict: {'mx304-11': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19'}
local_name: mx10008-23 Previous keyID: c107281c-9d04-4900-b731-896ab8f3a6c2
local_name: mx304-12: {'keys': [{'key_ID': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19', 'key': 'nK45tzFFjDA42/1Ko5B6Hz2wi35gr72VsiHyVbMD9X9qxRbHOKe8D4NpHUy1nX96JeVF4xOp0qOiDLH9ldIb4Q=='}]}
ckn abcd1234abcd5678abcd1234abcd5678d9b6fa63dd8847d28afe94fcecf40d19
cak 9cae39b731458c3038dbfd4aa3907a1f3db08b7e60afbd95b221f255b303f57f
local_name: mx304-12 commit True
Configuration <configuration>
  <security>
    <macsec>
      <connectivity-association>
        <name>CA_basic</name>
        <cipher-suite>gcm-aes-xpn-256</cipher-suite>
        <security-mode>static-cak</security-mode>
        <pre-shared-key>
          <ckn>abcd1234abcd5678abcd1234abcd5678d9b6fa63dd8847d28afe94fcecf40d19</ckn>
          <cak>9cae39b731458c3038dbfd4aa3907a1f3db08b7e60afbd95b221f255b303f57f</cak>
        </pre-shared-key>
      </connectivity-association>
    </macsec>
  </security>
</configuration>

local_name: mx10008-24: {'keys': [{'key_ID': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19', 'key': 'nK45tzFFjDA42/1Ko5B6Hz2wi35gr72VsiHyVbMD9X9qxRbHOKe8D4NpHUy1nX96JeVF4xOp0qOiDLH9ldIb4Q=='}]}
ckn abcd1234abcd5678abcd1234abcd5678d9b6fa63dd8847d28afe94fcecf40d19
cak 9cae39b731458c3038dbfd4aa3907a1f3db08b7e60afbd95b221f255b303f57f
local_name: mx10008-24 commit True
Configuration <configuration>
  <security>
    <macsec>
      <connectivity-association>
        <name>CA_basic</name>
        <cipher-suite>gcm-aes-xpn-256</cipher-suite>
        <security-mode>static-cak</security-mode>
        <pre-shared-key>
          <ckn>abcd1234abcd5678abcd1234abcd5678d9b6fa63dd8847d28afe94fcecf40d19</ckn>
          <cak>9cae39b731458c3038dbfd4aa3907a1f3db08b7e60afbd95b221f255b303f57f</cak>
        </pre-shared-key>
      </connectivity-association>
    </macsec>
  </security>
</configuration>

local_name: mx10008-23: {'keys': [{'key_ID': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19', 'key': 'nK45tzFFjDA42/1Ko5B6Hz2wi35gr72VsiHyVbMD9X9qxRbHOKe8D4NpHUy1nX96JeVF4xOp0qOiDLH9ldIb4Q=='}]}
ckn abcd1234abcd5678abcd1234abcd5678d9b6fa63dd8847d28afe94fcecf40d19
cak 9cae39b731458c3038dbfd4aa3907a1f3db08b7e60afbd95b221f255b303f57f
local_name: mx10008-23 commit True
Configuration <configuration>
  <security>
    <macsec>
      <connectivity-association>
        <name>CA_basic</name>
        <cipher-suite>gcm-aes-xpn-256</cipher-suite>
        <security-mode>static-cak</security-mode>
        <pre-shared-key>
          <ckn>abcd1234abcd5678abcd1234abcd5678d9b6fa63dd8847d28afe94fcecf40d19</ckn>
          <cak>9cae39b731458c3038dbfd4aa3907a1f3db08b7e60afbd95b221f255b303f57f</cak>
        </pre-shared-key>
      </connectivity-association>
    </macsec>
  </security>
</configuration>

========================== script run SUCCESS =============================
{'mx10008-23': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19'}
Saved the key IDs to the JSON file: /home/administrator/mx10008-23_last_key_test.json
========================== script run SUCCESS =============================
{'mx10008-24': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19'}
Saved the key IDs to the JSON file: /home/administrator/mx10008-24_last_key_test.json
========================== script run SUCCESS =============================
{'mx304-12': 'd9b6fa63-dd88-47d2-8afe-94fcecf40d19'}
Saved the key IDs to the JSON file: /home/administrator/mx304-12_last_key_test.json
----------------------mx304-11-----------------------------
<output>
Interface    Member-ID                  Type        Status        Tx        Rx        CAK Name
et-0/0/9     A2FC7A578AC21E943BF48DBC   primary     live          8         4         ABCD1234ABCD5678ABCD1234ABCD5678D9B6FA63DD8847D28AFE94FCECF40D19
xe-0/0/1:0   F6AC6383308556191AAFB283   primary     live          9         7         ABCD1234ABCD5678ABCD1234ABCD5678D9B6FA63DD8847D28AFE94FCECF40D19
</output>

stop process
----------------------mx10008-23-----------------------------
<output>
Interface    Member-ID                  Type        Status        Tx        Rx        CAK Name
et-0/0/1     7EA175A5FCE62846080918AD   primary     live          9         8         ABCD1234ABCD5678ABCD1234ABCD5678D9B6FA63DD8847D28AFE94FCECF40D19
xe-0/0/2:0   3E1BF61913EDC22757C9F013   primary     live          9         9         ABCD1234ABCD5678ABCD1234ABCD5678D9B6FA63DD8847D28AFE94FCECF40D19
</output>

stop process
----------------------mx10008-24-----------------------------
<output>
Interface    Member-ID                  Type        Status        Tx        Rx        CAK Name
et-0/0/1     75146F34B0738685954F1E48   primary     live          9         8         ABCD1234ABCD5678ABCD1234ABCD5678D9B6FA63DD8847D28AFE94FCECF40D19
</output>

stop process
----------------------mx304-12-----------------------------
<output>
Interface    Member-ID                  Type        Status        Tx        Rx        CAK Name
et-0/0/9     535246D1CF595CCD1BA8B067   primary     live          9         10        ABCD1234ABCD5678ABCD1234ABCD5678D9B6FA63DD8847D28AFE94FCECF40D19
</output>

stop process

### Onbox execution master

- Master device under test: mx304-11

ahanciu@mx304-11-re0> op ETSIA_v3.0.py    
onbox True
Logging modules initialized successfully
Onbox approach taken
local_name: mx304-11
mx304-11 is Master
/var/home/remote/mx304-11.crt
/var/home/remote/mx304-11.key
/var/home/remote/account-1286-server-ca-qukaydee-com.crt
local_name: mx304-11: {'keys': [{'key_ID': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9', 'key': 'E/n0dB6HY4PfpoYvJOuaEHba9PhB1/hz+zl1B6Wh71jkcBql9VayN1rFigJqw0R8IWiej8IxkDnDP/8Ofkw1rA=='}]}
inside try
KME: [GET] Get Keys API: https://kme-5.acct-1286.etsi-qkd-api.qukaydee.com/api/v1/keys/mx304-12/enc_keys
KME: [GET] Get Keys API: for mx304-12 {'keys': [{'key_ID': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9', 'key': 'E/n0dB6HY4PfpoYvJOuaEHba9PhB1/hz+zl1B6Wh71jkcBql9VayN1rFigJqw0R8IWiej8IxkDnDP/8Ofkw1rA=='}]}
Received KeyId:a310923e-9de9-4ab1-bafe-806bcc2e62f9
ckn abcd1234abcd5678abcd1234abcd5678a310923e9de94ab1bafe806bcc2e62f9
cak 13f9f4741e876383dfa6862f24eb9a1076daf4f841d7f873fb397507a5a1ef58
local_name: mx304-11 commit True
Configuration <configuration>
  <security>
    <macsec><connectivity-association>
                <name>CA_basic</name>
                <cipher-suite>gcm-aes-xpn-256</cipher-suite>
                <security-mode>static-cak</security-mode>
                <pre-shared-key>
                    <ckn>abcd1234abcd5678abcd1234abcd5678a310923e9de94ab1bafe806bcc2e62f9</ckn>
                    <cak>13f9f4741e876383dfa6862f24eb9a1076daf4f841d7f873fb397507a5a1ef58</cak>
                </pre-shared-key>
            </connectivity-association>
            </macsec>
  </security>
</configuration>
{'mx304-11': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9'}
Saved the key IDs to the JSON file: /var/home/remote/mx304-11_last_key_test.json
----------------------mx304-11-----------------------------
<output>
Interface    Member-ID                  Type        Status        Tx        Rx        CAK Name
et-0/0/9     F3793517620D59C1F98119D6   primary     live          43        39        ABCD1234ABCD5678ABCD1234ABCD5678FF6A6454A59B4A11A3F267F397398BEF
et-0/0/9     98DD8DE595C05146185D6C85   primary     in-progress   2         0         ABCD1234ABCD5678ABCD1234ABCD5678A310923E9DE94AB1BAFE806BCC2E62F9
xe-0/0/1:0   08A726358F84ACB1F0C99FE0   primary     live          43        36        ABCD1234ABCD5678ABCD1234ABCD5678FF6A6454A59B4A11A3F267F397398BEF
xe-0/0/1:0   E6388930FE1A773944C09EBA   primary     in-progress   2         0         ABCD1234ABCD5678ABCD1234ABCD5678A310923E9DE94AB1BAFE806BCC2E62F9
</output>
========================== script run SUCCESS =============================

### Onbox execution slave1

- Slave device under test: mx304-12

ahanciu@mx304-12-re0> op ETSIA_v3.0.py    
onbox True
Logging modules initialized successfully
Onbox approach taken
local_name: mx304-12
mx304-12 is Slave
local_name: mx304-12 last_key_dict: {'mx304-12': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx304-12 master_key_dict: {'mx304-11': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx304-12 Same KeyId: ff6a6454-a59b-4a11-a3f2-67f397398bef, Retrying 1
local_name: mx304-12 last_key_dict: {'mx304-12': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx304-12 master_key_dict: {'mx304-11': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx304-12 Same KeyId: ff6a6454-a59b-4a11-a3f2-67f397398bef, Retrying 2
local_name: mx304-12 last_key_dict: {'mx304-12': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx304-12 master_key_dict: {'mx304-11': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx304-12 Same KeyId: ff6a6454-a59b-4a11-a3f2-67f397398bef, Retrying 3
local_name: mx304-12 last_key_dict: {'mx304-12': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx304-12 master_key_dict: {'mx304-11': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx304-12 Same KeyId: ff6a6454-a59b-4a11-a3f2-67f397398bef, Retrying 4
local_name: mx304-12 last_key_dict: {'mx304-12': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx304-12 master_key_dict: {'mx304-11': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9'}
local_name: mx304-12 Previous keyID: ff6a6454-a59b-4a11-a3f2-67f397398bef
local_name: mx304-12: {'keys': [{'key_ID': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9', 'key': 'E/n0dB6HY4PfpoYvJOuaEHba9PhB1/hz+zl1B6Wh71jkcBql9VayN1rFigJqw0R8IWiej8IxkDnDP/8Ofkw1rA=='}]}
ckn abcd1234abcd5678abcd1234abcd5678a310923e9de94ab1bafe806bcc2e62f9
cak 13f9f4741e876383dfa6862f24eb9a1076daf4f841d7f873fb397507a5a1ef58
local_name: mx304-12 commit True
Configuration <configuration>
  <security>
    <macsec><connectivity-association>
                <name>CA_basic</name>
                <cipher-suite>gcm-aes-xpn-256</cipher-suite>
                <security-mode>static-cak</security-mode>
                <pre-shared-key>
                    <ckn>abcd1234abcd5678abcd1234abcd5678a310923e9de94ab1bafe806bcc2e62f9</ckn>
                    <cak>13f9f4741e876383dfa6862f24eb9a1076daf4f841d7f873fb397507a5a1ef58</cak>
                </pre-shared-key>
            </connectivity-association>
            </macsec>
  </security>
</configuration>
========================== script run SUCCESS =============================
{'mx304-12': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9'}
Saved the key IDs to the JSON file: /var/home/remote/mx304-12_last_key_test.json
----------------------mx304-12-----------------------------
<output>
Interface    Member-ID                  Type        Status        Tx        Rx        CAK Name
et-0/0/9     FCC2076AA69DCBAE2011CC38   primary     live          44        46        ABCD1234ABCD5678ABCD1234ABCD5678FF6A6454A59B4A11A3F267F397398BEF
et-0/0/9     5DD52C660EA70BD7DE81F439   primary     in-progress   2         0         ABCD1234ABCD5678ABCD1234ABCD5678A310923E9DE94AB1BAFE806BCC2E62F9
</output>

### Onbox execution additional slave2

- Additional slave device under test: mx10008-23

ahanciu@mx10008-23-re0> op ETSIA_v3.0.py    
onbox True
Logging modules initialized successfully
Onbox approach taken
local_name: mx10008-23
mx10008-23 is Slave
mx10008-23
{'mx10008-23': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 last_key_dict: {'mx10008-23': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 master_key_dict: {'mx304-11': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 Same KeyId: ff6a6454-a59b-4a11-a3f2-67f397398bef, Retrying 1
mx10008-23
{'mx10008-23': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 last_key_dict: {'mx10008-23': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 master_key_dict: {'mx304-11': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 Same KeyId: ff6a6454-a59b-4a11-a3f2-67f397398bef, Retrying 2
mx10008-23
{'mx10008-23': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 last_key_dict: {'mx10008-23': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 master_key_dict: {'mx304-11': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 Same KeyId: ff6a6454-a59b-4a11-a3f2-67f397398bef, Retrying 3
mx10008-23
{'mx10008-23': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 last_key_dict: {'mx10008-23': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-23 master_key_dict: {'mx304-11': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9'}
local_name: mx10008-23 Previous keyID: ff6a6454-a59b-4a11-a3f2-67f397398bef
local_name: mx10008-23: {'keys': [{'key_ID': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9', 'key': 'E/n0dB6HY4PfpoYvJOuaEHba9PhB1/hz+zl1B6Wh71jkcBql9VayN1rFigJqw0R8IWiej8IxkDnDP/8Ofkw1rA=='}]}
ckn abcd1234abcd5678abcd1234abcd5678a310923e9de94ab1bafe806bcc2e62f9
cak 13f9f4741e876383dfa6862f24eb9a1076daf4f841d7f873fb397507a5a1ef58
local_name: mx10008-23 commit True
Configuration <configuration>
  <security>
    <macsec><connectivity-association>
                <name>CA_basic</name>
                <cipher-suite>gcm-aes-xpn-256</cipher-suite>
                <security-mode>static-cak</security-mode>
                <pre-shared-key>
                    <ckn>abcd1234abcd5678abcd1234abcd5678a310923e9de94ab1bafe806bcc2e62f9</ckn>
                    <cak>13f9f4741e876383dfa6862f24eb9a1076daf4f841d7f873fb397507a5a1ef58</cak>
                </pre-shared-key>
            </connectivity-association>
            </macsec>
  </security>
</configuration>
========================== script run SUCCESS =============================
{'mx10008-23': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9'}
Saved the key IDs to the JSON file: /var/home/remote/mx10008-23_last_key_test.json
----------------------mx10008-23-----------------------------
<output>
Interface    Member-ID                  Type        Status        Tx        Rx        CAK Name
et-0/0/1     9D553DC27696177B2E5649A4   primary     live          38        23        ABCD1234ABCD5678ABCD1234ABCD5678FF6A6454A59B4A11A3F267F397398BEF
et-0/0/1     E5F4FC6970435F42E1AA8924   primary     in-progress   2         0         ABCD1234ABCD5678ABCD1234ABCD5678A310923E9DE94AB1BAFE806BCC2E62F9
xe-0/0/2:0   9D0DA77F1FFC68249674AA7C   primary     live          38        43        ABCD1234ABCD5678ABCD1234ABCD5678FF6A6454A59B4A11A3F267F397398BEF
xe-0/0/2:0   BCC4E0D4221BC1B34ADB2ABB   primary     in-progress   2         0         ABCD1234ABCD5678ABCD1234ABCD5678A310923E9DE94AB1BAFE806BCC2E62F9
</output>

### Onbox execution additional slave3

- Additional slave device under test: mx10008-24

ahanciu@mx10008-24> op ETSIA_v3.0.py    
onbox True
Logging modules initialized successfully
Onbox approach taken
local_name: mx10008-24
mx10008-24 is Slave
local_name: mx10008-24 last_key_dict: {'mx10008-24': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-24 master_key_dict: {'mx304-11': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-24 Same KeyId: ff6a6454-a59b-4a11-a3f2-67f397398bef, Retrying 1
local_name: mx10008-24 last_key_dict: {'mx10008-24': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-24 master_key_dict: {'mx304-11': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-24 Same KeyId: ff6a6454-a59b-4a11-a3f2-67f397398bef, Retrying 2
local_name: mx10008-24 last_key_dict: {'mx10008-24': 'ff6a6454-a59b-4a11-a3f2-67f397398bef'}
local_name: mx10008-24 master_key_dict: {'mx304-11': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9'}
local_name: mx10008-24 Previous keyID: ff6a6454-a59b-4a11-a3f2-67f397398bef
local_name: mx10008-24: {'keys': [{'key_ID': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9', 'key': 'E/n0dB6HY4PfpoYvJOuaEHba9PhB1/hz+zl1B6Wh71jkcBql9VayN1rFigJqw0R8IWiej8IxkDnDP/8Ofkw1rA=='}]}
ckn abcd1234abcd5678abcd1234abcd5678a310923e9de94ab1bafe806bcc2e62f9
cak 13f9f4741e876383dfa6862f24eb9a1076daf4f841d7f873fb397507a5a1ef58
local_name: mx10008-24 commit True
Configuration <configuration>
  <security>
    <macsec><connectivity-association>
                <name>CA_basic</name>
                <cipher-suite>gcm-aes-xpn-256</cipher-suite>
                <security-mode>static-cak</security-mode>
                <pre-shared-key>
                    <ckn>abcd1234abcd5678abcd1234abcd5678a310923e9de94ab1bafe806bcc2e62f9</ckn>
                    <cak>13f9f4741e876383dfa6862f24eb9a1076daf4f841d7f873fb397507a5a1ef58</cak>
                </pre-shared-key>
            </connectivity-association>
            </macsec>
  </security>
</configuration>
========================== script run SUCCESS =============================
{'mx10008-24': 'a310923e-9de9-4ab1-bafe-806bcc2e62f9'}
Saved the key IDs to the JSON file: /var/home/remote/mx10008-24_last_key_test.json
----------------------mx10008-24-----------------------------
<output>
Interface    Member-ID                  Type        Status        Tx        Rx        CAK Name
et-0/0/1     D5C45816AF10660C0DBF9F04   primary     live          23        36        ABCD1234ABCD5678ABCD1234ABCD5678FF6A6454A59B4A11A3F267F397398BEF
et-0/0/1     3E0D5101F36A32C97ABE2CD9   primary     in-progress   2         0         ABCD1234ABCD5678ABCD1234ABCD5678A310923E9DE94AB1BAFE806BCC2E62F9
</output>